1	Basic Call Between two UAs on the same server	2

2	Calls between two phones on different proxies	10

3	Multiple endpoints and forking	20

4	Canceling a call	30

5	Call Hold	37

6	Blind Transfer	51

7	Call with Authentication	69

8	Call Conference	82

# Basic Call Between two UAs on the same server

This call flow shows a successful call between two phones and a single proxy.

Figure 1: Calls Between UAs on the Same Server

| 1. | Internet Protocol, Src Addr: 10.1.71.188 (10.1.71.188), Dst Addr: 10.1.70.249 (10.1.70.249) User Datagram Protocol, Src Port: 5060 (5060), Dst Port: 5060 (5060) Session Initiation Protocol Request line: INVITE sip:pat@10.1.70.249:5060 SIP/2.0 Message Header From: "ray" <sip:ray@zultys.com:5060>;tag=zip4x4_2230727446-110 To: "pat" <sip:pat@10.1.70.249:5060> Call-ID: 2230726953-110 CSeq: 1 INVITE Via: SIP/2.0/UDP 10.1.71.188:5060 Contact: ray <sip:ray@10.1.71.188:5060> Allow: INVITE,BYE,CANCEL,ACK,OPTIONS,REGISTER,NOTIFY,MESSAGE,REFER User-Agent: Zultys ZIP4x4 1.1.60 Content-Type: application/sdp Content-Length: 259 Session Description Protocol Session Description Protocol Version (v): 0 Owner/Creator, Session Id (o): ZIP4x4-ray-1.1.60 0 0 IN IP4 10.1.71.188 Session Name (s): zultys media Connection Information (c): IN IP4 10.1.71.188 Time Description, active time (t): 0 0 Media Description, name and address (m): audio 20000 RTP/AVP 18 0 8 101 Media Attribute (a): rtpmap:0 PCMU/8000 Media Attribute (a): rtpmap:8 PCMA/8000 Media Attribute (a): rtpmap:18 G729/8000 Media Attribute (a): rtpmap:101 telephone-event/8000 Media Attribute (a): fmtp:101 0-15 Media Attribute (a): ptime:20 |
| --- | --- |
| 2. | Internet Protocol, Src Addr: 10.1.70.249 (10.1.70.249), Dst Addr: 10.1.71.188 (10.1.71.188) User Datagram Protocol, Src Port: 5060 (5060), Dst Port: 5060 (5060) Session Initiation Protocol Status line: SIP/2.0 100 trying -- your call is important to us Message Header From: "ray" <sip:ray@zultys.com:5060>;tag=zip4x4_2230727446-110 To: "pat" <sip:pat@10.1.70.249:5060> Call-ID: 2230726953-110 CSeq: 1 INVITE Via: SIP/2.0/UDP 10.1.71.188:5060 Server: Sip EXpress router (0.8.12 (i386/linux)) Content-Length: 0 Warning: 392 10.1.70.249:5060 "Noisy feedback tells: pid=3842 req_src_ip=10.1.71.188 req_src_port=5060 in_uri=sip:pat@10.1.70.249:5060 out_uri=sip:pat@10.1.71.187:5060 via_cnt==1" |
| 3. | Internet Protocol, Src Addr: 10.1.70.249 (10.1.70.249), Dst Addr: 10.1.71.187 (10.1.71.187) User Datagram Protocol, Src Port: 5060 (5060), Dst Port: 5060 (5060) Session Initiation Protocol Request line: INVITE sip:pat@10.1.71.187:5060 SIP/2.0 Message Header Max-Forwards: 10 Record-Route: <sip:pat@10.1.70.249;ftag=zip4x4_2230727446-110;lr=on> From: "ray" <sip:ray@zultys.com:5060>;tag=zip4x4_2230727446-110 To: "pat" <sip:pat@10.1.70.249:5060> Call-ID: 2230726953-110 CSeq: 1 INVITE Via: SIP/2.0/UDP 10.1.70.249;branch=z9hG4bKa227.6d384486.0 Via: SIP/2.0/UDP 10.1.71.188:5060 Contact: ray <sip:ray@10.1.71.188:5060> Allow: INVITE,BYE,CANCEL,ACK,OPTIONS,REGISTER,NOTIFY,MESSAGE,REFER User-Agent: Zultys ZIP4x4 1.1.60 Content-Type: application/sdp Content-Length: 259 P-hint: USRLOC Session Description Protocol Session Description Protocol Version (v): 0 Owner/Creator, Session Id (o): ZIP4x4-ray-1.1.60 0 0 IN IP4 10.1.71.188 Session Name (s): zultys media Connection Information (c): IN IP4 10.1.71.188 Time Description, active time (t): 0 0 Media Description, name and address (m): audio 20000 RTP/AVP 18 0 8 101 Media Attribute (a): rtpmap:0 PCMU/8000 Media Attribute (a): rtpmap:8 PCMA/8000 Media Attribute (a): rtpmap:18 G729/8000 Media Attribute (a): rtpmap:101 telephone-event/8000 Media Attribute (a): fmtp:101 0-15 Media Attribute (a): ptime:20 |
| 4. | Internet Protocol, Src Addr: 10.1.71.187 (10.1.71.187), Dst Addr: 10.1.70.249 (10.1.70.249) User Datagram Protocol, Src Port: 5060 (5060), Dst Port: 5060 (5060) Session Initiation Protocol Status line: SIP/2.0 100 Trying Message Header From: "ray" <sip:ray@zultys.com:5060>;tag=zip4x4_2230727446-110 To: "pat" <sip:pat@10.1.70.249:5060>;tag=zip4x4_2004527748-42 Call-ID: 2230726953-110 CSeq: 1 INVITE Via: SIP/2.0/UDP 10.1.70.249;branch=z9hG4bKa227.6d384486.0,SIP/2.0/UDP 10.1.71.188:5060 Record-Route: <sip:pat@10.1.70.249;ftag=zip4x4_2230727446-110;lr=on> Contact: pat <sip:pat@10.1.71.187:5060> Content-Length: 0 |
| 5. | Internet Protocol, Src Addr: 10.1.71.187 (10.1.71.187), Dst Addr: 10.1.70.249 (10.1.70.249) User Datagram Protocol, Src Port: 5060 (5060), Dst Port: 5060 (5060) Session Initiation Protocol Status line: SIP/2.0 180 Ringing Message Header From: "ray" <sip:ray@zultys.com:5060>;tag=zip4x4_2230727446-110 To: "pat" <sip:pat@10.1.70.249:5060>;tag=zip4x4_2004527748-42 Call-ID: 2230726953-110 CSeq: 1 INVITE Via: SIP/2.0/UDP 10.1.70.249;branch=z9hG4bKa227.6d384486.0,SIP/2.0/UDP 10.1.71.188:5060 Record-Route: <sip:pat@10.1.70.249;ftag=zip4x4_2230727446-110;lr=on> Contact: pat <sip:pat@10.1.71.187:5060> Content-Length: 0 |
| 6. | Internet Protocol, Src Addr: 10.1.70.249 (10.1.70.249), Dst Addr: 10.1.71.188 (10.1.71.188) User Datagram Protocol, Src Port: 5060 (5060), Dst Port: 5060 (5060) Session Initiation Protocol Status line: SIP/2.0 180 Ringing Message Header From: "ray" <sip:ray@zultys.com:5060>;tag=zip4x4_2230727446-110 To: "pat" <sip:pat@10.1.70.249:5060>;tag=zip4x4_2004527748-42 Call-ID: 2230726953-110 CSeq: 1 INVITE Via: SIP/2.0/UDP 10.1.71.188:5060 Record-Route: <sip:pat@10.1.70.249;ftag=zip4x4_2230727446-110;lr=on> Contact: pat <sip:pat@10.1.71.187:5060> Content-Length: 0 |
| 7. | Internet Protocol, Src Addr: 10.1.71.187 (10.1.71.187), Dst Addr: 10.1.70.249 (10.1.70.249) User Datagram Protocol, Src Port: 5060 (5060), Dst Port: 5060 (5060) Session Initiation Protocol Status line: SIP/2.0 200 OK Message Header From: "ray" <sip:ray@zultys.com:5060>;tag=zip4x4_2230727446-110 To: "pat" <sip:pat@10.1.70.249:5060>;tag=zip4x4_2004527748-42 Call-ID: 2230726953-110 CSeq: 1 INVITE Via: SIP/2.0/UDP 10.1.70.249;branch=z9hG4bKa227.6d384486.0,SIP/2.0/UDP 10.1.71.188:5060 Record-Route: <sip:pat@10.1.70.249;ftag=zip4x4_2230727446-110;lr=on> Contact: pat <sip:pat@10.1.71.187:5060> Allow: INVITE,BYE,CANCEL,ACK,OPTIONS,REGISTER,NOTIFY,MESSAGE,REFER Content-Type: application/sdp Content-Length: 199 Session Description Protocol Session Description Protocol Version (v): 0 Owner/Creator, Session Id (o): ZIP4x4-pat-1.2.14 0 0 IN IP4 10.1.71.187 Session Name (s): zultys media Connection Information (c): IN IP4 10.1.71.187 Time Description, active time (t): 0 0 Media Description, name and address (m): audio 33000 RTP/AVP 18 101 Media Attribute (a): rtpmap:18 G729/8000 Media Attribute (a): rtpmap:101 telephone-event/8000 Media Attribute (a): fmtp:101 0-15 |
| 8. | Internet Protocol, Src Addr: 10.1.70.249 (10.1.70.249), Dst Addr: 10.1.71.188 (10.1.71.188) User Datagram Protocol, Src Port: 5060 (5060), Dst Port: 5060 (5060) Session Initiation Protocol Status line: SIP/2.0 200 OK Message Header From: "ray" <sip:ray@zultys.com:5060>;tag=zip4x4_2230727446-110 To: "pat" <sip:pat@10.1.70.249:5060>;tag=zip4x4_2004527748-42 Call-ID: 2230726953-110 CSeq: 1 INVITE Via: SIP/2.0/UDP 10.1.71.188:5060 Record-Route: <sip:pat@10.1.70.249;ftag=zip4x4_2230727446-110;lr=on> Contact: pat <sip:pat@10.1.71.187:5060> Allow: INVITE,BYE,CANCEL,ACK,OPTIONS,REGISTER,NOTIFY,MESSAGE,REFER Content-Type: application/sdp Content-Length: 199 Session Description Protocol Session Description Protocol Version (v): 0 Owner/Creator, Session Id (o): ZIP4x4-pat-1.2.14 0 0 IN IP4 10.1.71.187 Session Name (s): zultys media Connection Information (c): IN IP4 10.1.71.187 Time Description, active time (t): 0 0 Media Description, name and address (m): audio 33000 RTP/AVP 18 101 Media Attribute (a): rtpmap:18 G729/8000 Media Attribute (a): rtpmap:101 telephone-event/8000 Media Attribute (a): fmtp:101 0-15 |
| 9. | Internet Protocol, Src Addr: 10.1.71.188 (10.1.71.188), Dst Addr: 10.1.70.249 (10.1.70.249) User Datagram Protocol, Src Port: 5060 (5060), Dst Port: 5060 (5060) Session Initiation Protocol Request line: ACK sip:pat@10.1.70.249:5060 SIP/2.0 Message Header From: "ray" <sip:ray@zultys.com:5060>;tag=zip4x4_2230727446-110 To: "pat" <sip:pat@10.1.70.249:5060>;tag=zip4x4_2004527748-42 Call-ID: 2230726953-110 CSeq: 1 ACK Via: SIP/2.0/UDP 10.1.71.188:5060 Contact: ray <sip:ray@10.1.71.188:5060> Route: <sip:pat@10.1.70.249;ftag=zip4x4_2230727446-110;lr=on>,<sip:pat@10.1.71.187:5060> Content-Length: 0 |
| 10. | Internet Protocol, Src Addr: 10.1.70.249 (10.1.70.249), Dst Addr: 10.1.71.187 (10.1.71.187) User Datagram Protocol, Src Port: 5060 (5060), Dst Port: 5060 (5060) Session Initiation Protocol Request line: ACK sip:pat@10.1.71.187:5060 SIP/2.0 Message Header Max-Forwards: 10 Record-Route: <sip:pat@10.1.70.249;ftag=zip4x4_2230727446-110;lr=on> From: "ray" <sip:ray@zultys.com:5060>;tag=zip4x4_2230727446-110 To: "pat" <sip:pat@10.1.70.249:5060>;tag=zip4x4_2004527748-42 Call-ID: 2230726953-110 CSeq: 1 ACK Via: SIP/2.0/UDP 10.1.70.249;branch=0 Via: SIP/2.0/UDP 10.1.71.188:5060 Contact: ray <sip:ray@10.1.71.188:5060> Content-Length: 0 |
| 11. | Internet Protocol, Src Addr: 10.1.71.187 (10.1.71.187), Dst Addr: 10.1.70.249 (10.1.70.249) User Datagram Protocol, Src Port: 5060 (5060), Dst Port: 5060 (5060) Session Initiation Protocol Request line: BYE sip:pat@10.1.70.249;ftag=zip4x4_2230727446-110;lr=on SIP/2.0 Message Header From: "pat" <sip:pat@10.1.70.249:5060>;tag=zip4x4_2004527748-42 To: "ray" <sip:ray@zultys.com:5060>;tag=zip4x4_2230727446-110 Call-ID: 2230726953-110 CSeq: 2 BYE Via: SIP/2.0/UDP 10.1.71.187:5060 Route: <sip:ray@10.1.71.188:5060> Content-Length: 0 |
| 12. | Internet Protocol, Src Addr: 10.1.70.249 (10.1.70.249), Dst Addr: 10.1.71.188 (10.1.71.188) User Datagram Protocol, Src Port: 5060 (5060), Dst Port: 5060 (5060) Session Initiation Protocol Request line: BYE sip:ray@10.1.71.188:5060 SIP/2.0 Message Header Max-Forwards: 10 Record-Route: <sip:pat@10.1.70.249;ftag=zip4x4_2004527748-42;lr=on> From: "pat" <sip:pat@10.1.70.249:5060>;tag=zip4x4_2004527748-42 To: "ray" <sip:ray@zultys.com:5060>;tag=zip4x4_2230727446-110 Call-ID: 2230726953-110 CSeq: 2 BYE Via: SIP/2.0/UDP 10.1.70.249;branch=z9hG4bK7227.bf81bbd3.0 Via: SIP/2.0/UDP 10.1.71.187:5060 Content-Length: 0 |
| 13. | Internet Protocol, Src Addr: 10.1.71.188 (10.1.71.188), Dst Addr: 10.1.70.249 (10.1.70.249) User Datagram Protocol, Src Port: 5060 (5060), Dst Port: 5060 (5060) Session Initiation Protocol Status line: SIP/2.0 200 OK Message Header From: "pat" <sip:pat@10.1.70.249:5060>;tag=zip4x4_2004527748-42 To: "ray" <sip:ray@zultys.com:5060>;tag=zip4x4_2230727446-110 Call-ID: 2230726953-110 CSeq: 2 BYE Via: SIP/2.0/UDP 10.1.70.249;branch=z9hG4bK7227.bf81bbd3.0,SIP/2.0/UDP 10.1.71.187:5060 Record-Route: <sip:pat@10.1.70.249;ftag=zip4x4_2004527748-42;lr=on> Contact: ray <sip:ray@10.1.71.188:5060> Allow: INVITE,BYE,CANCEL,ACK,OPTIONS,REGISTER,NOTIFY,MESSAGE,REFER Content-Length: 0 |
| 14. | Internet Protocol, Src Addr: 10.1.70.249 (10.1.70.249), Dst Addr: 10.1.71.187 (10.1.71.187) User Datagram Protocol, Src Port: 5060 (5060), Dst Port: 5060 (5060) Session Initiation Protocol Status line: SIP/2.0 200 OK Message Header From: "pat" <sip:pat@10.1.70.249:5060>;tag=zip4x4_2004527748-42 To: "ray" <sip:ray@zultys.com:5060>;tag=zip4x4_2230727446-110 Call-ID: 2230726953-110 CSeq: 2 BYE Via: SIP/2.0/UDP 10.1.71.187:5060 Record-Route: <sip:pat@10.1.70.249;ftag=zip4x4_2004527748-42;lr=on> Contact: ray <sip:ray@10.1.71.188:5060> Allow: INVITE,BYE,CANCEL,ACK,OPTIONS,REGISTER,NOTIFY,MESSAGE,REFER Content-Length: 0 |

# Calls between two phones on different proxies

This diagram shown below in Figure 2 shows a call flow for a call between two phones on different proxies. Here the IP addresses of the proxies are known. The endpoints are ZIP 4x4 IP phones. The proxy with IP address 10.1.40.25 is a Dynamicsoft proxy and the proxy with IP address 10.1.70.249 is a SER. If a call is destined for a second proxy for which the IP address is not known, DNS-SRV can be used to look up the appropriate destination proxy and then the name can be resolved to an IP address.

In step 5, notice that the SER inserts a Record-Route header. This causes the BYE sent in step 16 to go to the SER rather than directly to the other endpoint. Note that the Dynamicsoft did not insert this header so the BYE is sent from the SER to the other endpoint.

Figure 2: Call between UAs on different proxy servers

| 1. | Internet Protocol, Src Addr: 10.1.71.179 (10.1.71.179), Dst Addr: SIP (10.1.40.25) User Datagram Protocol, Src Port: 5060 (5060), Dst Port: 5060 (5060) Session Initiation Protocol Request line: INVITE sip:005@10.1.70.249:5060 SIP/2.0 Message Header From: "ZIP4_2" <sip:ZIP4_2@10.1.40.25:5060>;tag=zip4x4_3822292296-144 To: "005" <sip:005@10.1.70.249:5060> Call-ID: 3822291857-144 CSeq: 1 INVITE Via: SIP/2.0/UDP 10.1.71.179:5060 Contact: ZIP4_2 <sip:ZIP4_2@10.1.71.179:5060> Allow: INVITE,BYE,CANCEL,ACK,OPTIONS,REGISTER,NOTIFY,MESSAGE,REFER User-Agent: Zultys ZIP4x4 1.1.60 Content-Type: application/sdp Content-Length: 262 Session Description Protocol Session Description Protocol Version (v): 0 Owner/Creator, Session Id (o): ZIP4x4-ZIP4_2-1.1.60 0 0 IN IP4 10.1.71.179 Session Name (s): zultys media Connection Information (c): IN IP4 10.1.71.179 Time Description, active time (t): 0 0 Media Description, name and address (m): audio 20000 RTP/AVP 18 0 8 101 Media Attribute (a): rtpmap:0 PCMU/8000 Media Attribute (a): rtpmap:8 PCMA/8000 Media Attribute (a): rtpmap:18 G729/8000 Media Attribute (a): rtpmap:101 telephone-event/8000 Media Attribute (a): fmtp:101 0-15 Media Attribute (a): ptime:20 |
| --- | --- |
| 2. | Internet Protocol, Src Addr: SIP (10.1.40.25), Dst Addr: 10.1.71.179 (10.1.71.179) User Datagram Protocol, Src Port: 4247 (4247), Dst Port: 5060 (5060) Session Initiation Protocol Status line: SIP/2.0 100 Trying Message Header Via: SIP/2.0/UDP 10.1.71.179:5060 From: "ZIP4_2" <sip:ZIP4_2@10.1.40.25:5060>;tag=zip4x4_3822292296-144 To: "005" <sip:005@10.1.70.249:5060> Call-ID: 3822291857-144 CSeq: 1 INVITE Content-Length: 0 |
| 3. | Internet Protocol, Src Addr: SIP (10.1.40.25), Dst Addr: 10.1.70.249 (10.1.70.249) User Datagram Protocol, Src Port: 4248 (4248), Dst Port: 5060 (5060) Session Initiation Protocol Request line: INVITE sip:005@10.1.70.249:5060 SIP/2.0 Message Header From: "ZIP4_2" <sip:ZIP4_2@10.1.40.25:5060>;tag=zip4x4_3822292296-144 To: "005" <sip:005@10.1.70.249:5060> Call-ID: 3822291857-144 CSeq: 1 INVITE Via: SIP/2.0/UDP 10.1.40.25:5060;branch=1_90VOtx1a8a5pAeevvAK3rmw99 Via: SIP/2.0/UDP 10.1.71.179:5060 Contact:ZIP4_2 <sip:ZIP4_2@10.1.71.179:5060> Allow:INVITE,BYE,CANCEL,ACK,OPTIONS,REGISTER,NOTIFY,MESSAGE,REFER User-Agent:Zultys ZIP4x4 1.1.60 Content-Type:application/sdp Content-Length: 262 Session Description Protocol Session Description Protocol Version (v): 0 Owner/Creator, Session Id (o): ZIP4x4-ZIP4_2-1.1.60 0 0 IN IP4 10.1.71.179 Session Name (s): zultys media Connection Information (c): IN IP4 10.1.71.179 Time Description, active time (t): 0 0 Media Description, name and address (m): audio 20000 RTP/AVP 18 0 8 101 Media Attribute (a): rtpmap:0 PCMU/8000 Media Attribute (a): rtpmap:8 PCMA/8000 Media Attribute (a): rtpmap:18 G729/8000 Media Attribute (a): rtpmap:101 telephone-event/8000 Media Attribute (a): fmtp:101 0-15 Media Attribute (a): ptime:20 |
| 4. | Internet Protocol, Src Addr: 10.1.70.249 (10.1.70.249), Dst Addr: SIP (10.1.40.25) User Datagram Protocol, Src Port: 5060 (5060), Dst Port: 5060 (5060) Session Initiation Protocol Status line: SIP/2.0 100 trying -- your call is important to us Message Header From: "ZIP4_2" <sip:ZIP4_2@10.1.40.25:5060>;tag=zip4x4_3822292296-144 To: "005" <sip:005@10.1.70.249:5060> Call-ID: 3822291857-144 CSeq: 1 INVITE Via: SIP/2.0/UDP 10.1.40.25:5060;branch=1_90VOtx1a8a5pAeevvAK3rmw99 Via: SIP/2.0/UDP 10.1.71.179:5060 Server: Sip EXpress router (0.8.12 (i386/linux)) Content-Length: 0 Warning: 392 10.1.70.249:5060 "Noisy feedback tells: pid=20600 req_src_ip=10.1.40.25 req_src_port=4248 in_uri=sip:005@10.1.70.249:5060 out_uri=sip:pat@10.1.71.187:5060 via_cnt==2" |
| 5. | Internet Protocol, Src Addr: 10.1.70.249 (10.1.70.249), Dst Addr: 10.1.71.187 (10.1.71.187) User Datagram Protocol, Src Port: 5060 (5060), Dst Port: 5060 (5060) Session Initiation Protocol Request line: INVITE sip:pat@10.1.71.187:5060 SIP/2.0 Message Header Max-Forwards: 10 Record-Route: <sip:005@10.1.70.249;ftag=zip4x4_3822292296-144;lr=on> From: "ZIP4_2" <sip:ZIP4_2@10.1.40.25:5060>;tag=zip4x4_3822292296-144 To: "005" <sip:005@10.1.70.249:5060> Call-ID: 3822291857-144 CSeq: 1 INVITE Via: SIP/2.0/UDP 10.1.70.249;branch=z9hG4bKfa1e.4f84c3d3.0 Via: SIP/2.0/UDP 10.1.40.25:5060;branch=1_90VOtx1a8a5pAeevvAK3rmw99 Via: SIP/2.0/UDP 10.1.71.179:5060 Contact:ZIP4_2 <sip:ZIP4_2@10.1.71.179:5060> Allow:INVITE,BYE,CANCEL,ACK,OPTIONS,REGISTER,NOTIFY,MESSAGE,REFER User-Agent:Zultys ZIP4x4 1.1.60 Content-Type:application/sdp Content-Length: 262 Session Description Protocol Session Description Protocol Version (v): 0 Owner/Creator, Session Id (o): ZIP4x4-ZIP4_2-1.1.60 0 0 IN IP4 10.1.71.179 Session Name (s): zultys media Connection Information (c): IN IP4 10.1.71.179 Time Description, active time (t): 0 0 Media Description, name and address (m): audio 20000 RTP/AVP 18 0 8 101 Media Attribute (a): rtpmap:0 PCMU/8000 Media Attribute (a): rtpmap:8 PCMA/8000 Media Attribute (a): rtpmap:18 G729/8000 Media Attribute (a): rtpmap:101 telephone-event/8000 Media Attribute (a): fmtp:101 0-15 Media Attribute (a): ptime:20 |
| 6. | Internet Protocol, Src Addr: 10.1.71.187 (10.1.71.187), Dst Addr: 10.1.70.249 (10.1.70.249) User Datagram Protocol, Src Port: 5060 (5060), Dst Port: 5060 (5060) Session Initiation Protocol Status line: SIP/2.0 100 Trying Message Header From: "ZIP4_2" <sip:ZIP4_2@10.1.40.25:5060>;tag=zip4x4_3822292296-144 To: "005" <sip:005@10.1.70.249:5060>;tag=zip4x4_2918767198-42 Call-ID: 3822291857-144 CSeq: 1 INVITE Via: SIP/2.0/UDP 10.1.70.249;branch=z9hG4bKfa1e.4f84c3d3.0,SIP/2.0/UDP 10.1.40.25:5060;branch=1_90VOtx1a8a5pAeevvAK3rmw99,SIP/2.0/UDP 10.1.71.179:5060 Record-Route: <sip:005@10.1.70.249;ftag=zip4x4_3822292296-144;lr=on> Contact: pat <sip:pat@10.1.71.187:5060> Content-Length: 0 |
| 7. | Internet Protocol, Src Addr: 10.1.71.187 (10.1.71.187), Dst Addr: 10.1.70.249 (10.1.70.249) User Datagram Protocol, Src Port: 5060 (5060), Dst Port: 5060 (5060) Session Initiation Protocol Status line: SIP/2.0 180 Ringing Message Header From: "ZIP4_2" <sip:ZIP4_2@10.1.40.25:5060>;tag=zip4x4_3822292296-144 To: "005" <sip:005@10.1.70.249:5060>;tag=zip4x4_2918767198-42 Call-ID: 3822291857-144 CSeq: 1 INVITE Via: SIP/2.0/UDP 10.1.70.249;branch=z9hG4bKfa1e.4f84c3d3.0,SIP/2.0/UDP 10.1.40.25:5060;branch=1_90VOtx1a8a5pAeevvAK3rmw99,SIP/2.0/UDP 10.1.71.179:5060 Record-Route: <sip:005@10.1.70.249;ftag=zip4x4_3822292296-144;lr=on> Contact: pat <sip:pat@10.1.71.187:5060> Content-Length: 0 |
| 8. | Internet Protocol, Src Addr: 10.1.70.249 (10.1.70.249), Dst Addr: SIP (10.1.40.25) User Datagram Protocol, Src Port: 5060 (5060), Dst Port: 5060 (5060) Session Initiation Protocol Status line: SIP/2.0 180 Ringing Message Header From: "ZIP4_2" <sip:ZIP4_2@10.1.40.25:5060>;tag=zip4x4_3822292296-144 To: "005" <sip:005@10.1.70.249:5060>;tag=zip4x4_2918767198-42 Call-ID: 3822291857-144 CSeq: 1 INVITE Via: SIP/2.0/UDP 10.1.40.25:5060;branch=1_90VOtx1a8a5pAeevvAK3rmw99,SIP/2.0/UDP 10.1.71.179:5060 Record-Route: <sip:005@10.1.70.249;ftag=zip4x4_3822292296-144;lr=on> Contact: pat <sip:pat@10.1.71.187:5060> Content-Length: 0 |
| 9. | Internet Protocol, Src Addr: SIP (10.1.40.25), Dst Addr: 10.1.71.179 (10.1.71.179) User Datagram Protocol, Src Port: 4247 (4247), Dst Port: 5060 (5060) Session Initiation Protocol Status line: SIP/2.0 180 Ringing Message Header From: "ZIP4_2" <sip:ZIP4_2@10.1.40.25:5060>;tag=zip4x4_3822292296-144 To: "005" <sip:005@10.1.70.249:5060>;tag=zip4x4_2918767198-42 Call-ID: 3822291857-144 CSeq: 1 INVITE Via: SIP/2.0/UDP 10.1.71.179:5060 Record-Route:<sip:005@10.1.70.249;ftag=zip4x4_3822292296-144;lr=on> Contact:pat <sip:pat@10.1.71.187:5060> Content-Length: 0 |
| 10. | Internet Protocol, Src Addr: 10.1.71.187 (10.1.71.187), Dst Addr: 10.1.70.249 (10.1.70.249) User Datagram Protocol, Src Port: 5060 (5060), Dst Port: 5060 (5060) Session Initiation Protocol Status line: SIP/2.0 200 OK Message Header From: "ZIP4_2" <sip:ZIP4_2@10.1.40.25:5060>;tag=zip4x4_3822292296-144 To: "005" <sip:005@10.1.70.249:5060>;tag=zip4x4_2918767198-42 Call-ID: 3822291857-144 CSeq: 1 INVITE Via: SIP/2.0/UDP 10.1.70.249;branch=z9hG4bKfa1e.4f84c3d3.0,SIP/2.0/UDP 10.1.40.25:5060;branch=1_90VOtx1a8a5pAeevvAK3rmw99,SIP/2.0/UDP 10.1.71.179:5060 Record-Route: <sip:005@10.1.70.249;ftag=zip4x4_3822292296-144;lr=on> Contact: pat <sip:pat@10.1.71.187:5060> Allow: INVITE,BYE,CANCEL,ACK,OPTIONS,REGISTER,NOTIFY,MESSAGE,REFER Content-Type: application/sdp Content-Length: 199 Session Description Protocol Session Description Protocol Version (v): 0 Owner/Creator, Session Id (o): ZIP4x4-pat-1.2.14 0 0 IN IP4 10.1.71.187 Session Name (s): zultys media Connection Information (c): IN IP4 10.1.71.187 Time Description, active time (t): 0 0 Media Description, name and address (m): audio 33000 RTP/AVP 18 101 Media Attribute (a): rtpmap:18 G729/8000 Media Attribute (a): rtpmap:101 telephone-event/8000 Media Attribute (a): fmtp:101 0-15 |
| 11. | Internet Protocol, Src Addr: 10.1.70.249 (10.1.70.249), Dst Addr: SIP (10.1.40.25) User Datagram Protocol, Src Port: 5060 (5060), Dst Port: 5060 (5060) Session Initiation Protocol Status line: SIP/2.0 200 OK Message Header From: "ZIP4_2" <sip:ZIP4_2@10.1.40.25:5060>;tag=zip4x4_3822292296-144 To: "005" <sip:005@10.1.70.249:5060>;tag=zip4x4_2918767198-42 Call-ID: 3822291857-144 CSeq: 1 INVITE Via: SIP/2.0/UDP 10.1.40.25:5060;branch=1_90VOtx1a8a5pAeevvAK3rmw99,SIP/2.0/UDP 10.1.71.179:5060 Record-Route: <sip:005@10.1.70.249;ftag=zip4x4_3822292296-144;lr=on> Contact: pat <sip:pat@10.1.71.187:5060> Allow: INVITE,BYE,CANCEL,ACK,OPTIONS,REGISTER,NOTIFY,MESSAGE,REFER Content-Type: application/sdp Content-Length: 199 Session Description Protocol Session Description Protocol Version (v): 0 Owner/Creator, Session Id (o): ZIP4x4-pat-1.2.14 0 0 IN IP4 10.1.71.187 Session Name (s): zultys media Connection Information (c): IN IP4 10.1.71.187 Time Description, active time (t): 0 0 Media Description, name and address (m): audio 33000 RTP/AVP 18 101 Media Attribute (a): rtpmap:18 G729/8000 Media Attribute (a): rtpmap:101 telephone-event/8000 Media Attribute (a): fmtp:101 0-15 |
| 12. | Internet Protocol, Src Addr: SIP (10.1.40.25), Dst Addr: 10.1.71.179 (10.1.71.179) User Datagram Protocol, Src Port: 4247 (4247), Dst Port: 5060 (5060) Session Initiation Protocol Status line: SIP/2.0 200 OK Message Header From: "ZIP4_2" <sip:ZIP4_2@10.1.40.25:5060>;tag=zip4x4_3822292296-144 To: "005" <sip:005@10.1.70.249:5060>;tag=zip4x4_2918767198-42 Call-ID: 3822291857-144 CSeq: 1 INVITE Via: SIP/2.0/UDP 10.1.71.179:5060 Record-Route:<sip:005@10.1.70.249;ftag=zip4x4_3822292296-144;lr=on> Contact:pat <sip:pat@10.1.71.187:5060> Allow:INVITE,BYE,CANCEL,ACK,OPTIONS,REGISTER,NOTIFY,MESSAGE,REFER Content-Type:application/sdp Content-Length: 199 Session Description Protocol Session Description Protocol Version (v): 0 Owner/Creator, Session Id (o): ZIP4x4-pat-1.2.14 0 0 IN IP4 10.1.71.187 Session Name (s): zultys media Connection Information (c): IN IP4 10.1.71.187 Time Description, active time (t): 0 0 Media Description, name and address (m): audio 33000 RTP/AVP 18 101 Media Attribute (a): rtpmap:18 G729/8000 Media Attribute (a): rtpmap:101 telephone-event/8000 Media Attribute (a): fmtp:101 0-15 |
| 13. | Internet Protocol, Src Addr: 10.1.71.179 (10.1.71.179), Dst Addr: SIP (10.1.40.25) User Datagram Protocol, Src Port: 5060 (5060), Dst Port: 5060 (5060) Session Initiation Protocol Request line: ACK sip:005@10.1.70.249:5060 SIP/2.0 Message Header From: "ZIP4_2" <sip:ZIP4_2@10.1.40.25:5060>;tag=zip4x4_3822292296-144 To: "005" <sip:005@10.1.70.249:5060>;tag=zip4x4_2918767198-42 Call-ID: 3822291857-144 CSeq: 1 ACK Via: SIP/2.0/UDP 10.1.71.179:5060 Contact: ZIP4_2 <sip:ZIP4_2@10.1.71.179:5060> Route: <sip:005@10.1.70.249;ftag=zip4x4_3822292296-144;lr=on>,<sip:pat@10.1.71.187:5060> Content-Length: 0 |
| 14. | Internet Protocol, Src Addr: SIP (10.1.40.25), Dst Addr: 10.1.70.249 (10.1.70.249) User Datagram Protocol, Src Port: 4248 (4248), Dst Port: 5060 (5060) Session Initiation Protocol Request line: ACK sip:005@10.1.70.249;ftag=zip4x4_3822292296-144;lr=on SIP/2.0 Message Header From: "ZIP4_2" <sip:ZIP4_2@10.1.40.25:5060>;tag=zip4x4_3822292296-144 To: "005" <sip:005@10.1.70.249:5060>;tag=zip4x4_2918767198-42 Call-ID: 3822291857-144 CSeq: 1 ACK Via: SIP/2.0/UDP 10.1.40.25:5060;branch=1_90VOtx1a8a5pAeevvAK3rmw99 Via: SIP/2.0/UDP 10.1.71.179:5060 Contact:ZIP4_2 <sip:ZIP4_2@10.1.71.179:5060> Route: <sip:pat@10.1.71.187:5060> Content-Length: 0 |
| 15. | Internet Protocol, Src Addr: 10.1.70.249 (10.1.70.249), Dst Addr: 10.1.71.187 (10.1.71.187) User Datagram Protocol, Src Port: 5060 (5060), Dst Port: 5060 (5060) Session Initiation Protocol Request line: ACK sip:pat@10.1.71.187:5060 SIP/2.0 Message Header Max-Forwards: 10 Record-Route: <sip:005@10.1.70.249;ftag=zip4x4_3822292296-144;lr=on> From: "ZIP4_2" <sip:ZIP4_2@10.1.40.25:5060>;tag=zip4x4_3822292296-144 To: "005" <sip:005@10.1.70.249:5060>;tag=zip4x4_2918767198-42 Call-ID: 3822291857-144 CSeq: 1 ACK Via: SIP/2.0/UDP 10.1.70.249;branch=0 Via: SIP/2.0/UDP 10.1.40.25:5060;branch=1_90VOtx1a8a5pAeevvAK3rmw99 Via: SIP/2.0/UDP 10.1.71.179:5060 Contact:ZIP4_2 <sip:ZIP4_2@10.1.71.179:5060> Content-Length: 0 |
| 16. | Internet Protocol, Src Addr: 10.1.71.187 (10.1.71.187), Dst Addr: 10.1.70.249 (10.1.70.249) User Datagram Protocol, Src Port: 5060 (5060), Dst Port: 5060 (5060) Session Initiation Protocol Request line: BYE sip:005@10.1.70.249;ftag=zip4x4_3822292296-144;lr=on SIP/2.0 Message Header From: "005" <sip:005@10.1.70.249:5060>;tag=zip4x4_2918767198-42 To: "ZIP4_2" <sip:ZIP4_2@10.1.40.25:5060>;tag=zip4x4_3822292296-144 Call-ID: 3822291857-144 CSeq: 2 BYE Via: SIP/2.0/UDP 10.1.71.187:5060 Route: <sip:ZIP4_2@10.1.71.179:5060> Content-Length: 0 |
| 17. | User Datagram Protocol, Src Port: 5060 (5060), Dst Port: 5060 (5060) Session Initiation Protocol Request line: BYE sip:ZIP4_2@10.1.71.179:5060 SIP/2.0 Message Header Max-Forwards: 10 Record-Route: <sip:005@10.1.70.249;ftag=zip4x4_2918767198-42;lr=on> From: "005" <sip:005@10.1.70.249:5060>;tag=zip4x4_2918767198-42 To: "ZIP4_2" <sip:ZIP4_2@10.1.40.25:5060>;tag=zip4x4_3822292296-144 Call-ID: 3822291857-144 CSeq: 2 BYE Via: SIP/2.0/UDP 10.1.70.249;branch=z9hG4bKca1e.10f7d057.0 Via: SIP/2.0/UDP 10.1.71.187:5060 Content-Length: 0 |
| 18. | Internet Protocol, Src Addr: 10.1.71.179 (10.1.71.179), Dst Addr: 10.1.70.249 (10.1.70.249) User Datagram Protocol, Src Port: 5060 (5060), Dst Port: 5060 (5060) Session Initiation Protocol Status line: SIP/2.0 200 OK Message Header From: "005" <sip:005@10.1.70.249:5060>;tag=zip4x4_2918767198-42 To: "ZIP4_2" <sip:ZIP4_2@10.1.40.25:5060>;tag=zip4x4_3822292296-144 Call-ID: 3822291857-144 CSeq: 2 BYE Via: SIP/2.0/UDP 10.1.70.249;branch=z9hG4bKca1e.10f7d057.0,SIP/2.0/UDP 10.1.71.187:5060 Record-Route: <sip:005@10.1.70.249;ftag=zip4x4_2918767198-42;lr=on> Contact: ZIP4_2 <sip:ZIP4_2@10.1.71.179:5060> Allow: INVITE,BYE,CANCEL,ACK,OPTIONS,REGISTER,NOTIFY,MESSAGE,REFER Content-Length: 0 |
| 19. | Internet Protocol, Src Addr: 10.1.70.249 (10.1.70.249), Dst Addr: 10.1.71.187 (10.1.71.187) User Datagram Protocol, Src Port: 5060 (5060), Dst Port: 5060 (5060) Session Initiation Protocol Status line: SIP/2.0 200 OK Message Header From: "005" <sip:005@10.1.70.249:5060>;tag=zip4x4_2918767198-42 To: "ZIP4_2" <sip:ZIP4_2@10.1.40.25:5060>;tag=zip4x4_3822292296-144 Call-ID: 3822291857-144 CSeq: 2 BYE Via: SIP/2.0/UDP 10.1.71.187:5060 Record-Route: <sip:005@10.1.70.249;ftag=zip4x4_2918767198-42;lr=on> Contact: ZIP4_2 <sip:ZIP4_2@10.1.71.179:5060> Allow: INVITE,BYE,CANCEL,ACK,OPTIONS,REGISTER,NOTIFY,MESSAGE,REFER Content-Length: 0 |

# Multiple endpoints and forking

When a user has multiple contacts registered with a proxy, the proxy can fork the INVITE request to those multiple contacts. As soon as the call is answered on one of the contacts, the other call legs are terminated.

Figure 3: Multiple endpoints and call forking

| 1. | Internet Protocol, Src Addr: 10.1.71.150 (10.1.71.150), Dst Addr: SIP (10.1.40.25) User Datagram Protocol, Src Port: 5060 (5060), Dst Port: 5060 (5060) Session Initiation Protocol Request line: INVITE sip:ZIP4_2@10.1.40.25:5060 SIP/2.0 Message Header From: "ZIP4_1" <sip:ZIP4_1@10.1.40.25:5060>;tag=zip4x4_853257988-183 To: "ZIP4_2" <sip:ZIP4_2@10.1.40.25:5060> Call-ID: 853257381-183 CSeq: 1 INVITE Via: SIP/2.0/UDP 10.1.71.150:5060 Contact: ZIP4_1 <sip:ZIP4_1@10.1.71.150:5060> Allow: INVITE,BYE,CANCEL,ACK,OPTIONS,REGISTER,NOTIFY,MESSAGE,REFER User-Agent: Zultys ZIP4x4 1.2.11 Content-Type: application/sdp Content-Length: 262 Session Description Protocol Session Description Protocol Version (v): 0 Owner/Creator, Session Id (o): ZIP4x4-ZIP4_1-1.2.11 0 0 IN IP4 10.1.71.150 Session Name (s): zultys media Connection Information (c): IN IP4 10.1.71.150 Time Description, active time (t): 0 0 Media Description, name and address (m): audio 19996 RTP/AVP 0 8 18 101 Media Attribute (a): rtpmap:0 PCMU/8000 Media Attribute (a): rtpmap:8 PCMA/8000 Media Attribute (a): rtpmap:18 G729/8000 Media Attribute (a): rtpmap:101 telephone-event/8000 Media Attribute (a): fmtp:101 0-15 Media Attribute (a): ptime:20 |
| --- | --- |
| 2. | Internet Protocol, Src Addr: SIP (10.1.40.25), Dst Addr: 10.1.71.150 (10.1.71.150) User Datagram Protocol, Src Port: 4275 (4275), Dst Port: 5060 (5060) Session Initiation Protocol Status line: SIP/2.0 100 Trying Message Header Via: SIP/2.0/UDP 10.1.71.150:5060 From: "ZIP4_1" <sip:ZIP4_1@10.1.40.25:5060>;tag=zip4x4_853257988-183 To: "ZIP4_2" <sip:ZIP4_2@10.1.40.25:5060> Call-ID: 853257381-183 CSeq: 1 INVITE Content-Length: 0 |
| 3. | Internet Protocol, Src Addr: SIP (10.1.40.25), Dst Addr: 10.1.71.152 (10.1.71.152) User Datagram Protocol, Src Port: 4276 (4276), Dst Port: 5060 (5060) Session Initiation Protocol Request line: INVITE sip:ZIP4_2@10.1.71.152 SIP/2.0 Message Header From: "ZIP4_1" <sip:ZIP4_1@10.1.40.25:5060>;tag=zip4x4_853257988-183 To: "ZIP4_2" <sip:ZIP4_2@10.1.40.25:5060> Call-ID: 853257381-183 CSeq: 1 INVITE Via: SIP/2.0/UDP 10.1.40.25:5060;branch=1_z3240D90U9zYg9zU2OQemEvWA99 Via: SIP/2.0/UDP 10.1.71.150:5060 Contact:ZIP4_1 <sip:ZIP4_1@10.1.71.150:5060> Allow:INVITE,BYE,CANCEL,ACK,OPTIONS,REGISTER,NOTIFY,MESSAGE,REFER User-Agent:Zultys ZIP4x4 1.2.11 Content-Type:application/sdp Content-Length: 262 Session Description Protocol Session Description Protocol Version (v): 0 Owner/Creator, Session Id (o): ZIP4x4-ZIP4_1-1.2.11 0 0 IN IP4 10.1.71.150 Session Name (s): zultys media Connection Information (c): IN IP4 10.1.71.150 Time Description, active time (t): 0 0 Media Description, name and address (m): audio 19996 RTP/AVP 0 8 18 101 Media Attribute (a): rtpmap:0 PCMU/8000 Media Attribute (a): rtpmap:8 PCMA/8000 Media Attribute (a): rtpmap:18 G729/8000 Media Attribute (a): rtpmap:101 telephone-event/8000 Media Attribute (a): fmtp:101 0-15 Media Attribute (a): ptime:20 |
| 4. | Internet Protocol, Src Addr: SIP (10.1.40.25), Dst Addr: 10.1.71.179 (10.1.71.179) User Datagram Protocol, Src Port: 4277 (4277), Dst Port: 5060 (5060) Session Initiation Protocol Request line: INVITE sip:ZIP4_2@10.1.71.179 SIP/2.0 Message Header From: "ZIP4_1" <sip:ZIP4_1@10.1.40.25:5060>;tag=zip4x4_853257988-183 To: "ZIP4_2" <sip:ZIP4_2@10.1.40.25:5060> Call-ID: 853257381-183 CSeq: 1 INVITE Via: SIP/2.0/UDP 10.1.40.25:5060;branch=2_z3240D90U9zYg9zU2OQemEvWA99 Via: SIP/2.0/UDP 10.1.71.150:5060 Contact:ZIP4_1 <sip:ZIP4_1@10.1.71.150:5060> Allow:INVITE,BYE,CANCEL,ACK,OPTIONS,REGISTER,NOTIFY,MESSAGE,REFER User-Agent:Zultys ZIP4x4 1.2.11 Content-Type:application/sdp Content-Length: 262 Session Description Protocol Session Description Protocol Version (v): 0 Owner/Creator, Session Id (o): ZIP4x4-ZIP4_1-1.2.11 0 0 IN IP4 10.1.71.150 Session Name (s): zultys media Connection Information (c): IN IP4 10.1.71.150 Time Description, active time (t): 0 0 Media Description, name and address (m): audio 19996 RTP/AVP 0 8 18 101 Media Attribute (a): rtpmap:0 PCMU/8000 Media Attribute (a): rtpmap:8 PCMA/8000 Media Attribute (a): rtpmap:18 G729/8000 Media Attribute (a): rtpmap:101 telephone-event/8000 Media Attribute (a): fmtp:101 0-15 Media Attribute (a): ptime:20 |
| 5. | Internet Protocol, Src Addr: 10.1.71.152 (10.1.71.152), Dst Addr: SIP (10.1.40.25) User Datagram Protocol, Src Port: 5060 (5060), Dst Port: 5060 (5060) Session Initiation Protocol Status line: SIP/2.0 100 Trying Message Header From: "ZIP4_1" <sip:ZIP4_1@10.1.40.25:5060>;tag=zip4x4_853257988-183 To: "ZIP4_2" <sip:ZIP4_2@10.1.40.25:5060>;tag=zip4x4_3303382298-104 Call-ID: 853257381-183 CSeq: 1 INVITE Via: SIP/2.0/UDP 10.1.40.25:5060;branch=1_z3240D90U9zYg9zU2OQemEvWA99,SIP/2.0/UDP 10.1.71.150:5060 Contact: ZIP4_2 <sip:ZIP4_2@10.1.71.152:5060> Content-Length: 0 |
| 6. | Internet Protocol, Src Addr: 10.1.71.152 (10.1.71.152), Dst Addr: SIP (10.1.40.25) User Datagram Protocol, Src Port: 5060 (5060), Dst Port: 5060 (5060) Session Initiation Protocol Status line: SIP/2.0 180 Ringing Message Header From: "ZIP4_1" <sip:ZIP4_1@10.1.40.25:5060>;tag=zip4x4_853257988-183 To: "ZIP4_2" <sip:ZIP4_2@10.1.40.25:5060>;tag=zip4x4_3303382298-104 Call-ID: 853257381-183 CSeq: 1 INVITE Via: SIP/2.0/UDP 10.1.40.25:5060;branch=1_z3240D90U9zYg9zU2OQemEvWA99,SIP/2.0/UDP 10.1.71.150:5060 Contact: ZIP4_2 <sip:ZIP4_2@10.1.71.152:5060> Content-Length: 0 |
| 7. | Internet Protocol, Src Addr: SIP (10.1.40.25), Dst Addr: 10.1.71.150 (10.1.71.150) User Datagram Protocol, Src Port: 4275 (4275), Dst Port: 5060 (5060) Session Initiation Protocol Status line: SIP/2.0 180 Ringing Message Header From: "ZIP4_1" <sip:ZIP4_1@10.1.40.25:5060>;tag=zip4x4_853257988-183 To: "ZIP4_2" <sip:ZIP4_2@10.1.40.25:5060>;tag=zip4x4_3303382298-104 Call-ID: 853257381-183 CSeq: 1 INVITE Via: SIP/2.0/UDP 10.1.71.150:5060 Contact:ZIP4_2 <sip:ZIP4_2@10.1.71.152:5060> Content-Length: 0 |
| 8. | Internet Protocol, Src Addr: 10.1.71.179 (10.1.71.179), Dst Addr: SIP (10.1.40.25) User Datagram Protocol, Src Port: 5060 (5060), Dst Port: 5060 (5060) Session Initiation Protocol Status line: SIP/2.0 100 Trying Message Header From: "ZIP4_1" <sip:ZIP4_1@10.1.40.25:5060>;tag=zip4x4_853257988-183 To: "ZIP4_2" <sip:ZIP4_2@10.1.40.25:5060>;tag=zip4x4_482588019-144 Call-ID: 853257381-183 CSeq: 1 INVITE Via: SIP/2.0/UDP 10.1.40.25:5060;branch=2_z3240D90U9zYg9zU2OQemEvWA99,SIP/2.0/UDP 10.1.71.150:5060 Contact: ZIP4_2 <sip:ZIP4_2@10.1.71.179:5060> Content-Length: 0 |
| 9. | Internet Protocol, Src Addr: 10.1.71.179 (10.1.71.179), Dst Addr: SIP (10.1.40.25) User Datagram Protocol, Src Port: 5060 (5060), Dst Port: 5060 (5060) Session Initiation Protocol Status line: SIP/2.0 180 Ringing Message Header From: "ZIP4_1" <sip:ZIP4_1@10.1.40.25:5060>;tag=zip4x4_853257988-183 To: "ZIP4_2" <sip:ZIP4_2@10.1.40.25:5060>;tag=zip4x4_482588019-144 Call-ID: 853257381-183 CSeq: 1 INVITE Via: SIP/2.0/UDP 10.1.40.25:5060;branch=2_z3240D90U9zYg9zU2OQemEvWA99,SIP/2.0/UDP 10.1.71.150:5060 Contact: ZIP4_2 <sip:ZIP4_2@10.1.71.179:5060> Content-Length: 0 |
| 10. | Internet Protocol, Src Addr: SIP (10.1.40.25), Dst Addr: 10.1.71.150 (10.1.71.150) User Datagram Protocol, Src Port: 4275 (4275), Dst Port: 5060 (5060) Session Initiation Protocol Status line: SIP/2.0 180 Ringing Message Header From: "ZIP4_1" <sip:ZIP4_1@10.1.40.25:5060>;tag=zip4x4_853257988-183 To: "ZIP4_2" <sip:ZIP4_2@10.1.40.25:5060>;tag=zip4x4_482588019-144 Call-ID: 853257381-183 CSeq: 1 INVITE Via: SIP/2.0/UDP 10.1.71.150:5060 Contact:ZIP4_2 <sip:ZIP4_2@10.1.71.179:5060> Content-Length: 0 |
| 11. | Internet Protocol, Src Addr: 10.1.71.152 (10.1.71.152), Dst Addr: SIP (10.1.40.25) User Datagram Protocol, Src Port: 5060 (5060), Dst Port: 5060 (5060) Session Initiation Protocol Status line: SIP/2.0 200 OK Message Header From: "ZIP4_1" <sip:ZIP4_1@10.1.40.25:5060>;tag=zip4x4_853257988-183 To: "ZIP4_2" <sip:ZIP4_2@10.1.40.25:5060>;tag=zip4x4_3303382298-104 Call-ID: 853257381-183 CSeq: 1 INVITE Via: SIP/2.0/UDP 10.1.40.25:5060;branch=1_z3240D90U9zYg9zU2OQemEvWA99,SIP/2.0/UDP 10.1.71.150:5060 Contact: ZIP4_2 <sip:ZIP4_2@10.1.71.152:5060> Allow: INVITE,BYE,CANCEL,ACK,OPTIONS,REGISTER,NOTIFY,MESSAGE,REFER Content-Type: application/sdp Content-Length: 212 Session Description Protocol Session Description Protocol Version (v): 0 Owner/Creator, Session Id (o): ZIP4x4-ZIP4_2-1.2.14 0 0 IN IP4 10.1.71.152 Session Name (s): zultys media Connection Information (c): IN IP4 10.1.71.152 Time Description, active time (t): 0 0 Media Description, name and address (m): audio 33000 RTP/AVP 0 101 Media Attribute (a): rtpmap:0 PCMU/8000 Media Attribute (a): ptime:20 Media Attribute (a): rtpmap:101 telephone-event/8000 Media Attribute (a): fmtp:101 0-15 |
| 12. | Internet Protocol, Src Addr: SIP (10.1.40.25), Dst Addr: 10.1.71.179 (10.1.71.179) User Datagram Protocol, Src Port: 4277 (4277), Dst Port: 5060 (5060) Session Initiation Protocol Request line: CANCEL sip:ZIP4_2@10.1.71.179 SIP/2.0 Message Header From: "ZIP4_1" <sip:ZIP4_1@10.1.40.25:5060>;tag=zip4x4_853257988-183 To: "ZIP4_2" <sip:ZIP4_2@10.1.40.25:5060> Content-Length: 0 Call-ID: 853257381-183 CSeq: 1 CANCEL Via: SIP/2.0/UDP 10.1.40.25:5060;branch=2_z3240D90U9zYg9zU2OQemEvWA99 |
| 13. | Internet Protocol, Src Addr: SIP (10.1.40.25), Dst Addr: 10.1.71.150 (10.1.71.150) User Datagram Protocol, Src Port: 4275 (4275), Dst Port: 5060 (5060) Session Initiation Protocol Status line: SIP/2.0 200 OK Message Header From: "ZIP4_1" <sip:ZIP4_1@10.1.40.25:5060>;tag=zip4x4_853257988-183 To: "ZIP4_2" <sip:ZIP4_2@10.1.40.25:5060>;tag=zip4x4_3303382298-104 Call-ID: 853257381-183 CSeq: 1 INVITE Via: SIP/2.0/UDP 10.1.71.150:5060 Contact:ZIP4_2 <sip:ZIP4_2@10.1.71.152:5060> Allow:INVITE,BYE,CANCEL,ACK,OPTIONS,REGISTER,NOTIFY,MESSAGE,REFER Content-Type:application/sdp Content-Length: 212 Session Description Protocol Session Description Protocol Version (v): 0 Owner/Creator, Session Id (o): ZIP4x4-ZIP4_2-1.2.14 0 0 IN IP4 10.1.71.152 Session Name (s): zultys media Connection Information (c): IN IP4 10.1.71.152 Time Description, active time (t): 0 0 Media Description, name and address (m): audio 33000 RTP/AVP 0 101 Media Attribute (a): rtpmap:0 PCMU/8000 Media Attribute (a): ptime:20 Media Attribute (a): rtpmap:101 telephone-event/8000 Media Attribute (a): fmtp:101 0-15 |
| 14. | Internet Protocol, Src Addr: 10.1.71.179 (10.1.71.179), Dst Addr: SIP (10.1.40.25) User Datagram Protocol, Src Port: 5060 (5060), Dst Port: 5060 (5060) Session Initiation Protocol Status line: SIP/2.0 200 OK Message Header From: "ZIP4_1" <sip:ZIP4_1@10.1.40.25:5060>;tag=zip4x4_853257988-183 To: "ZIP4_2" <sip:ZIP4_2@10.1.40.25:5060>;tag=zip4x4_482588019-144 Call-ID: 853257381-183 CSeq: 1 CANCEL Via: SIP/2.0/UDP 10.1.40.25:5060;branch=2_z3240D90U9zYg9zU2OQemEvWA99 Contact: ZIP4_2 <sip:ZIP4_2@10.1.71.179:5060> Allow: INVITE,BYE,CANCEL,ACK,OPTIONS,REGISTER,NOTIFY,MESSAGE,REFER Content-Length: 0 |
| 15. | Internet Protocol, Src Addr: 10.1.71.150 (10.1.71.150), Dst Addr: SIP (10.1.40.25) User Datagram Protocol, Src Port: 5060 (5060), Dst Port: 5060 (5060) Session Initiation Protocol Request line: ACK sip:ZIP4_2@10.1.40.25:5060 SIP/2.0 Message Header From: "ZIP4_1" <sip:ZIP4_1@10.1.40.25:5060>;tag=zip4x4_853257988-183 To: "ZIP4_2" <sip:ZIP4_2@10.1.40.25:5060>;tag=zip4x4_3303382298-104 Call-ID: 853257381-183 CSeq: 1 ACK Via: SIP/2.0/UDP 10.1.71.150:5060 Contact: ZIP4_1 <sip:ZIP4_1@10.1.71.150:5060> Content-Length: 0 |
| 16. | Internet Protocol, Src Addr: SIP (10.1.40.25), Dst Addr: 10.1.71.152 (10.1.71.152) User Datagram Protocol, Src Port: 4276 (4276), Dst Port: 5060 (5060) Session Initiation Protocol Request line: ACK sip:ZIP4_2@10.1.71.152:5060 SIP/2.0 Message Header From: "ZIP4_1" <sip:ZIP4_1@10.1.40.25:5060>;tag=zip4x4_853257988-183 To: "ZIP4_2" <sip:ZIP4_2@10.1.40.25:5060>;tag=zip4x4_3303382298-104 Call-ID: 853257381-183 CSeq: 1 ACK Via: SIP/2.0/UDP 10.1.40.25:5060;branch=1_z3240D90U9zYg9zU2OQemEvWA99 Via: SIP/2.0/UDP 10.1.71.150:5060 Contact:ZIP4_1 <sip:ZIP4_1@10.1.71.150:5060> Content-Length: 0 |
| 17. | Internet Protocol, Src Addr: 10.1.71.179 (10.1.71.179), Dst Addr: SIP (10.1.40.25) User Datagram Protocol, Src Port: 5060 (5060), Dst Port: 5060 (5060) Session Initiation Protocol Status line: SIP/2.0 487 Request Terminated Message Header From: "ZIP4_1" <sip:ZIP4_1@10.1.40.25:5060>;tag=zip4x4_853257988-183 To: "ZIP4_2" <sip:ZIP4_2@10.1.40.25:5060>;tag=zip4x4_482588019-144 Call-ID: 853257381-183 CSeq: 1 INVITE Via: SIP/2.0/UDP 10.1.40.25:5060;branch=2_z3240D90U9zYg9zU2OQemEvWA99,SIP/2.0/UDP 10.1.71.150:5060 Contact: ZIP4_2 <sip:ZIP4_2@10.1.71.179:5060> Content-Length: 0 |
| 18. | Internet Protocol, Src Addr: SIP (10.1.40.25), Dst Addr: 10.1.71.179 (10.1.71.179) User Datagram Protocol, Src Port: 4277 (4277), Dst Port: 5060 (5060) Session Initiation Protocol Request line: ACK sip:ZIP4_2@10.1.71.179 SIP/2.0 Message Header From: "ZIP4_1" <sip:ZIP4_1@10.1.40.25:5060>;tag=zip4x4_853257988-183 To: "ZIP4_2" <sip:ZIP4_2@10.1.40.25:5060>;tag=zip4x4_482588019-144 Call-ID: 853257381-183 CSeq: 1 ACK Content-Length: 0 Via: SIP/2.0/UDP 10.1.40.25:5060;branch=2_z3240D90U9zYg9zU2OQemEvWA99 |
| 19. | Internet Protocol, Src Addr: 10.1.71.152 (10.1.71.152), Dst Addr: SIP (10.1.40.25) User Datagram Protocol, Src Port: 5060 (5060), Dst Port: 5060 (5060) Session Initiation Protocol Request line: BYE sip:ZIP4_1@10.1.40.25:5060 SIP/2.0 Message Header From: "ZIP4_2" <sip:ZIP4_2@10.1.40.25:5060>;tag=zip4x4_3303382298-104 To: "ZIP4_1" <sip:ZIP4_1@10.1.40.25:5060>;tag=zip4x4_853257988-183 Call-ID: 853257381-183 CSeq: 2 BYE Via: SIP/2.0/UDP 10.1.71.152:5060 Content-Length: 0 |
| 20. | Internet Protocol, Src Addr: SIP (10.1.40.25), Dst Addr: 10.1.71.150 (10.1.71.150) User Datagram Protocol, Src Port: 4275 (4275), Dst Port: 5060 (5060) Session Initiation Protocol Request line: BYE sip:ZIP4_1@10.1.71.150 SIP/2.0 Message Header From: "ZIP4_2" <sip:ZIP4_2@10.1.40.25:5060>;tag=zip4x4_3303382298-104 To: "ZIP4_1" <sip:ZIP4_1@10.1.40.25:5060>;tag=zip4x4_853257988-183 Call-ID: 853257381-183 CSeq: 2 BYE Via: SIP/2.0/UDP 10.1.40.25:5060;branch=1_1TaoTXPDdJUNvwg6LKVRfQ99 Via: SIP/2.0/UDP 10.1.71.152:5060 Content-Length: 0 |
| 21. | Internet Protocol, Src Addr: 10.1.71.150 (10.1.71.150), Dst Addr: SIP (10.1.40.25) User Datagram Protocol, Src Port: 5060 (5060), Dst Port: 5060 (5060) Session Initiation Protocol Status line: SIP/2.0 200 OK Message Header From: "ZIP4_2" <sip:ZIP4_2@10.1.40.25:5060>;tag=zip4x4_3303382298-104 To: "ZIP4_1" <sip:ZIP4_1@10.1.40.25:5060>;tag=zip4x4_853257988-183 Call-ID: 853257381-183 CSeq: 2 BYE Via: SIP/2.0/UDP 10.1.40.25:5060;branch=1_1TaoTXPDdJUNvwg6LKVRfQ99,SIP/2.0/UDP 10.1.71.152:5060 Contact: ZIP4_1 <sip:ZIP4_1@10.1.71.150:5060> Allow: INVITE,BYE,CANCEL,ACK,OPTIONS,REGISTER,NOTIFY,MESSAGE,REFER Content-Length: 0 |
| 22. | Internet Protocol, Src Addr: SIP (10.1.40.25), Dst Addr: 10.1.71.152 (10.1.71.152) User Datagram Protocol, Src Port: 4276 (4276), Dst Port: 5060 (5060) Session Initiation Protocol Status line: SIP/2.0 200 OK Message Header From: "ZIP4_2" <sip:ZIP4_2@10.1.40.25:5060>;tag=zip4x4_3303382298-104 To: "ZIP4_1" <sip:ZIP4_1@10.1.40.25:5060>;tag=zip4x4_853257988-183 Call-ID: 853257381-183 CSeq: 2 BYE Via: SIP/2.0/UDP 10.1.71.152:5060 Contact:ZIP4_1 <sip:ZIP4_1@10.1.71.150:5060> Allow:INVITE,BYE,CANCEL,ACK,OPTIONS,REGISTER,NOTIFY,MESSAGE,REFER Content-Length: 0 |

# Canceling a call

When a call is terminated before the far end answers, the call flow appears as follows:

Figure 4: Call Cancel

| 1. | Internet Protocol, Src Addr: 10.1.71.187 (10.1.71.187), Dst Addr: 10.1.70.249 (10.1.70.249) User Datagram Protocol, Src Port: 5060 (5060), Dst Port: 5060 (5060) Session Initiation Protocol Request line: INVITE sip:mx25_1@10.1.70.249:5060 SIP/2.0 Message Header From: "pat" <sip:pat@zultys.com:5060>;tag=zip4x4_1283100398-42 To: "mx25_1" <sip:mx25_1@10.1.70.249:5060> Call-ID: 1283099973-42 CSeq: 1 INVITE Via: SIP/2.0/UDP 10.1.71.187:5060 Contact: pat <sip:pat@10.1.71.187:5060> Allow: INVITE,BYE,CANCEL,ACK,OPTIONS,REGISTER,NOTIFY,MESSAGE,REFER User-Agent: Zultys ZIP4x4 1.2.14 Content-Type: application/sdp Content-Length: 259 Session Description Protocol Session Description Protocol Version (v): 0 Owner/Creator, Session Id (o): ZIP4x4-pat-1.2.14 0 0 IN IP4 10.1.71.187 Session Name (s): zultys media Connection Information (c): IN IP4 10.1.71.187 Time Description, active time (t): 0 0 Media Description, name and address (m): audio 33000 RTP/AVP 0 8 18 101 Media Attribute (a): rtpmap:0 PCMU/8000 Media Attribute (a): rtpmap:8 PCMA/8000 Media Attribute (a): rtpmap:18 G729/8000 Media Attribute (a): rtpmap:101 telephone-event/8000 Media Attribute (a): fmtp:101 0-15 Media Attribute (a): ptime:20 |
| --- | --- |
| 2. | Internet Protocol, Src Addr: 10.1.70.249 (10.1.70.249), Dst Addr: 10.1.71.187 (10.1.71.187) User Datagram Protocol, Src Port: 5060 (5060), Dst Port: 5060 (5060) Session Initiation Protocol Status line: SIP/2.0 100 trying -- your call is important to us Message Header From: "pat" <sip:pat@zultys.com:5060>;tag=zip4x4_1283100398-42 To: "mx25_1" <sip:mx25_1@10.1.70.249:5060> Call-ID: 1283099973-42 CSeq: 1 INVITE Via: SIP/2.0/UDP 10.1.71.187:5060 Server: Sip EXpress router (0.8.12 (i386/linux)) Content-Length: 0 Warning: 392 10.1.70.249:5060 "Noisy feedback tells: pid=3842 req_src_ip=10.1.71.187 req_src_port=5060 in_uri=sip:mx25_1@10.1.70.249:5060 out_uri=sip:MX25_1@10.1.71.254:5060 via_cnt==1" |
| 3. | Internet Protocol, Src Addr: 10.1.70.249 (10.1.70.249), Dst Addr: 10.1.71.254 (10.1.71.254) User Datagram Protocol, Src Port: 5060 (5060), Dst Port: 5060 (5060) Session Initiation Protocol Request line: INVITE sip:MX25_1@10.1.71.254:5060 SIP/2.0 Message Header Max-Forwards: 10 Record-Route: <sip:mx25_1@10.1.70.249;ftag=zip4x4_1283100398-42;lr=on> From: "pat" <sip:pat@zultys.com:5060>;tag=zip4x4_1283100398-42 To: "mx25_1" <sip:mx25_1@10.1.70.249:5060> Call-ID: 1283099973-42 CSeq: 1 INVITE Via: SIP/2.0/UDP 10.1.70.249;branch=z9hG4bK9c5c.3fec73c4.0 Via: SIP/2.0/UDP 10.1.71.187:5060 Contact: pat <sip:pat@10.1.71.187:5060> Allow: INVITE,BYE,CANCEL,ACK,OPTIONS,REGISTER,NOTIFY,MESSAGE,REFER User-Agent: Zultys ZIP4x4 1.2.14 Content-Type: application/sdp Content-Length: 259 P-hint: USRLOC Session Description Protocol Session Description Protocol Version (v): 0 Owner/Creator, Session Id (o): ZIP4x4-pat-1.2.14 0 0 IN IP4 10.1.71.187 Session Name (s): zultys media Connection Information (c): IN IP4 10.1.71.187 Time Description, active time (t): 0 0 Media Description, name and address (m): audio 33000 RTP/AVP 0 8 18 101 Media Attribute (a): rtpmap:0 PCMU/8000 Media Attribute (a): rtpmap:8 PCMA/8000 Media Attribute (a): rtpmap:18 G729/8000 Media Attribute (a): rtpmap:101 telephone-event/8000 Media Attribute (a): fmtp:101 0-15 Media Attribute (a): ptime:20 |
| 4. | Internet Protocol, Src Addr: 10.1.71.254 (10.1.71.254), Dst Addr: 10.1.70.249 (10.1.70.249) User Datagram Protocol, Src Port: 5060 (5060), Dst Port: 5060 (5060) Session Initiation Protocol Status line: SIP/2.0 100 Trying Message Header From: "pat" <sip:pat@zultys.com:5060>;tag=zip4x4_1283100398-42 To: "mx25_1" <sip:mx25_1@10.1.70.249:5060>;tag=5-nvs--103599998392_1843337145-92 Call-ID: 1283099973-42 CSeq: 1 INVITE Via: SIP/2.0/UDP 10.1.70.249;branch=z9hG4bK9c5c.3fec73c4.0,SIP/2.0/UDP 10.1.71.187:5060 Record-Route: <sip:mx25_1@10.1.70.249;ftag=zip4x4_1283100398-42;lr=on> Server: Zultys MX25 Content-Length: 0 |
| 5. | Internet Protocol, Src Addr: 10.1.71.254 (10.1.71.254), Dst Addr: 10.1.70.249 (10.1.70.249) User Datagram Protocol, Src Port: 5060 (5060), Dst Port: 5060 (5060) Session Initiation Protocol Status line: SIP/2.0 180 Ringing Message Header From: "pat" <sip:pat@zultys.com:5060>;tag=zip4x4_1283100398-42 To: "mx25_1" <sip:mx25_1@10.1.70.249:5060>;tag=5-nvs--103599998392_1843337145-92 Call-ID: 1283099973-42 CSeq: 1 INVITE Via: SIP/2.0/UDP 10.1.70.249;branch=z9hG4bK9c5c.3fec73c4.0,SIP/2.0/UDP 10.1.71.187:5060 Record-Route: <sip:mx25_1@10.1.70.249;ftag=zip4x4_1283100398-42;lr=on> Server: Zultys MX25 Content-Length: 0 |
| 6. | Internet Protocol, Src Addr: 10.1.70.249 (10.1.70.249), Dst Addr: 10.1.71.187 (10.1.71.187) User Datagram Protocol, Src Port: 5060 (5060), Dst Port: 5060 (5060) Session Initiation Protocol Status line: SIP/2.0 180 Ringing Message Header From: "pat" <sip:pat@zultys.com:5060>;tag=zip4x4_1283100398-42 To: "mx25_1" <sip:mx25_1@10.1.70.249:5060>;tag=5-nvs--103599998392_1843337145-92 Call-ID: 1283099973-42 CSeq: 1 INVITE Via: SIP/2.0/UDP 10.1.71.187:5060 Record-Route: <sip:mx25_1@10.1.70.249;ftag=zip4x4_1283100398-42;lr=on> Server: Zultys MX25 Content-Length: 0 |
| 7. | Internet Protocol, Src Addr: 10.1.71.187 (10.1.71.187), Dst Addr: 10.1.70.249 (10.1.70.249) User Datagram Protocol, Src Port: 5060 (5060), Dst Port: 5060 (5060) Session Initiation Protocol Request line: CANCEL sip:mx25_1@10.1.70.249:5060 SIP/2.0 Message Header From: "pat" <sip:pat@zultys.com:5060>;tag=zip4x4_1283100398-42 To: "mx25_1" <sip:mx25_1@10.1.70.249:5060> Call-ID: 1283099973-42 CSeq: 1 CANCEL Via: SIP/2.0/UDP 10.1.71.187:5060 Content-Length: 0 |
| 8. | Internet Protocol, Src Addr: 10.1.70.249 (10.1.70.249), Dst Addr: 10.1.71.254 (10.1.71.254) User Datagram Protocol, Src Port: 5060 (5060), Dst Port: 5060 (5060) Session Initiation Protocol Request line: CANCEL sip:MX25_1@10.1.71.254:5060 SIP/2.0 Message Header Max-Forwards: 10 Record-Route: <sip:mx25_1@10.1.70.249;ftag=zip4x4_1283100398-42;lr=on> From: "pat" <sip:pat@zultys.com:5060>;tag=zip4x4_1283100398-42 To: "mx25_1" <sip:mx25_1@10.1.70.249:5060> Call-ID: 1283099973-42 CSeq: 1 CANCEL Via: SIP/2.0/UDP 10.1.70.249;branch=z9hG4bK9c5c.3fec73c4.0 Via: SIP/2.0/UDP 10.1.71.187:5060 Content-Length: 0 P-hint: USRLOC |
| 9. | Internet Protocol, Src Addr: 10.1.70.249 (10.1.70.249), Dst Addr: 10.1.71.187 (10.1.71.187) User Datagram Protocol, Src Port: 5060 (5060), Dst Port: 5060 (5060) Session Initiation Protocol Status line: SIP/2.0 200 cancelling Message Header From: "pat" <sip:pat@zultys.com:5060>;tag=zip4x4_1283100398-42 To: "mx25_1" <sip:mx25_1@10.1.70.249:5060>;tag=d033ce68bf1b8d5eb856789c2809f286-1e83 Call-ID: 1283099973-42 CSeq: 1 CANCEL Via: SIP/2.0/UDP 10.1.71.187:5060 Server: Sip EXpress router (0.8.12 (i386/linux)) Content-Length: 0 Warning: 392 10.1.70.249:5060 "Noisy feedback tells: pid=3842 req_src_ip=10.1.71.187 req_src_port=5060 in_uri=sip:mx25_1@10.1.70.249:5060 out_uri=sip:MX25_1@10.1.71.254:5060 via_cnt==1" |
| 10. | Internet Protocol, Src Addr: 10.1.70.249 (10.1.70.249), Dst Addr: 10.1.71.187 (10.1.71.187) User Datagram Protocol, Src Port: 5060 (5060), Dst Port: 5060 (5060) Session Initiation Protocol Status line: SIP/2.0 487 Request cancelled Message Header From: "pat" <sip:pat@zultys.com:5060>;tag=zip4x4_1283100398-42 To: "mx25_1" <sip:mx25_1@10.1.70.249:5060>;tag=d033ce68bf1b8d5eb856789c2809f286-1e83 Call-ID: 1283099973-42 CSeq: 1 INVITE Via: SIP/2.0/UDP 10.1.71.187:5060 Server: Sip EXpress router (0.8.12 (i386/linux)) Content-Length: 0 Warning: 392 10.1.70.249:5060 "Noisy feedback tells: pid=3842 req_src_ip=10.1.71.187 req_src_port=5060 in_uri=sip:mx25_1@10.1.70.249:5060 out_uri=sip:MX25_1@10.1.71.254:5060 via_cnt==1" |
| 11. | Internet Protocol, Src Addr: 10.1.71.254 (10.1.71.254), Dst Addr: 10.1.70.249 (10.1.70.249) User Datagram Protocol, Src Port: 5060 (5060), Dst Port: 5060 (5060) Session Initiation Protocol Status line: SIP/2.0 200 OK Message Header From: "pat" <sip:pat@zultys.com:5060>;tag=zip4x4_1283100398-42 To: "mx25_1" <sip:mx25_1@10.1.70.249:5060>;tag=5-nvs--103599998392_1843337145-92 Call-ID: 1283099973-42 CSeq: 1 CANCEL Via: SIP/2.0/UDP 10.1.70.249;branch=z9hG4bK9c5c.3fec73c4.0,SIP/2.0/UDP 10.1.71.187:5060 Record-Route: <sip:mx25_1@10.1.70.249;ftag=zip4x4_1283100398-42;lr=on> Contact: sip:mx25_1@10.1.71.254:5060 Allow: INVITE,ACK,CANCEL,BYE,REGISTER,OPTIONS,NOTIFY,SUBSCRIBE,REFER,MESSAGE Server: Zultys MX25 Content-Length: 0 |
| 12. | Internet Protocol, Src Addr: 10.1.71.254 (10.1.71.254), Dst Addr: 10.1.70.249 (10.1.70.249) User Datagram Protocol, Src Port: 5060 (5060), Dst Port: 5060 (5060) Session Initiation Protocol Status line: SIP/2.0 487 Request Terminated Message Header From: "pat" <sip:pat@zultys.com:5060>;tag=zip4x4_1283100398-42 To: "mx25_1" <sip:mx25_1@10.1.70.249:5060>;tag=5-nvs--103599998392_1843337145-92 Call-ID: 1283099973-42 CSeq: 1 INVITE Via: SIP/2.0/UDP 10.1.70.249;branch=z9hG4bK9c5c.3fec73c4.0,SIP/2.0/UDP 10.1.71.187:5060 Record-Route: <sip:mx25_1@10.1.70.249;ftag=zip4x4_1283100398-42;lr=on> Contact: sip:mx25_1@10.1.71.254:5060 Server: Zultys MX25 Content-Length: 0 |
| 13. | Internet Protocol, Src Addr: 10.1.70.249 (10.1.70.249), Dst Addr: 10.1.71.254 (10.1.71.254) User Datagram Protocol, Src Port: 5060 (5060), Dst Port: 5060 (5060) Session Initiation Protocol Request line: ACK sip:MX25_1@10.1.71.254:5060 SIP/2.0 Message Header Via: SIP/2.0/UDP 10.1.70.249;branch=z9hG4bK9c5c.3fec73c4.0 From: "pat" <sip:pat@zultys.com:5060>;tag=zip4x4_1283100398-42 Call-ID: 1283099973-42 To: "mx25_1" <sip:mx25_1@10.1.70.249:5060>;tag=5-nvs--103599998392_1843337145-92 CSeq: 1 ACK User-Agent: Sip EXpress router(0.8.12 (i386/linux)) Content-Length: 0 |
| 14. | Internet Protocol, Src Addr: 10.1.71.187 (10.1.71.187), Dst Addr: 10.1.70.249 (10.1.70.249) User Datagram Protocol, Src Port: 5060 (5060), Dst Port: 5060 (5060) Session Initiation Protocol Request line: ACK sip:mx25_1@10.1.70.249:5060 SIP/2.0 Message Header From: "pat" <sip:pat@zultys.com:5060>;tag=zip4x4_1283100398-42 To: "mx25_1" <sip:mx25_1@10.1.70.249:5060>;tag=d033ce68bf1b8d5eb856789c2809f286-1e83 Call-ID: 1283099973-42 CSeq: 1 ACK Via: SIP/2.0/UDP 10.1.71.187:5060 Contact: pat <sip:pat@10.1.71.187:5060> Content-Length: 0 |

# Call Hold

This call trace shown in Figure 5 indicates how a SIP call is placed on hold.  In step 11 the phone indicates in the SDP that the connection information is IN IP4 0.0.0.0. This stops media from being sent to the previous address. This is one approach to placing a call on hold. Another approach is to use a=sendonly or a=inactive within the SDP.

Figure 5: Call Hold

| 1. | Internet Protocol, Src Addr: 10.1.71.152 (10.1.71.152), Dst Addr: SIP (10.1.40.25) User Datagram Protocol, Src Port: 5060 (5060), Dst Port: 5060 (5060) Session Initiation Protocol Request line: INVITE sip:ZIP4_1@10.1.40.25:5060 SIP/2.0 Message Header From: "ZIP4_2" <sip:ZIP4_2@10.1.40.25:5060>;tag=zip4x4_3571429956-104 To: "ZIP4_1" <sip:ZIP4_1@10.1.40.25:5060> Call-ID: 3571429538-104 CSeq: 1 INVITE Via: SIP/2.0/UDP 10.1.71.152:5060 Contact: ZIP4_2 <sip:ZIP4_2@10.1.71.152:5060> Allow: INVITE,BYE,CANCEL,ACK,OPTIONS,REGISTER,NOTIFY,MESSAGE,REFER User-Agent: Zultys ZIP4x4 1.2.14 Content-Type: application/sdp Content-Length: 262 Session Description Protocol Session Description Protocol Version (v): 0 Owner/Creator, Session Id (o): ZIP4x4-ZIP4_2-1.2.14 0 0 IN IP4 10.1.71.152 Session Name (s): zultys media Connection Information (c): IN IP4 10.1.71.152 Time Description, active time (t): 0 0 Media Description, name and address (m): audio 33000 RTP/AVP 0 8 18 101 Media Attribute (a): rtpmap:0 PCMU/8000 Media Attribute (a): rtpmap:8 PCMA/8000 Media Attribute (a): rtpmap:18 G729/8000 Media Attribute (a): rtpmap:101 telephone-event/8000 Media Attribute (a): fmtp:101 0-15 Media Attribute (a): ptime:20 |
| --- | --- |
| 2. | Internet Protocol, Src Addr: SIP (10.1.40.25), Dst Addr: 10.1.71.152 (10.1.71.152) User Datagram Protocol, Src Port: 4283 (4283), Dst Port: 5060 (5060) Session Initiation Protocol Status line: SIP/2.0 100 Trying Message Header Via: SIP/2.0/UDP 10.1.71.152:5060 From: "ZIP4_2" <sip:ZIP4_2@10.1.40.25:5060>;tag=zip4x4_3571429956-104 To: "ZIP4_1" <sip:ZIP4_1@10.1.40.25:5060> Call-ID: 3571429538-104 CSeq: 1 INVITE Content-Length: 0 |
| 3. | Internet Protocol, Src Addr: SIP (10.1.40.25), Dst Addr: 10.1.71.150 (10.1.71.150) User Datagram Protocol, Src Port: 4284 (4284), Dst Port: 5060 (5060) Session Initiation Protocol Request line: INVITE sip:ZIP4_1@10.1.71.150 SIP/2.0 Message Header From: "ZIP4_2" <sip:ZIP4_2@10.1.40.25:5060>;tag=zip4x4_3571429956-104 To: "ZIP4_1" <sip:ZIP4_1@10.1.40.25:5060> Call-ID: 3571429538-104 CSeq: 1 INVITE Via: SIP/2.0/UDP 10.1.40.25:5060;branch=1_1TaoTXPDdJUNvwg6LKVRfQ99 Via: SIP/2.0/UDP 10.1.71.152:5060 Contact:ZIP4_2 <sip:ZIP4_2@10.1.71.152:5060> Allow:INVITE,BYE,CANCEL,ACK,OPTIONS,REGISTER,NOTIFY,MESSAGE,REFER User-Agent:Zultys ZIP4x4 1.2.14 Content-Type:application/sdp Content-Length: 262 Session Description Protocol Session Description Protocol Version (v): 0 Owner/Creator, Session Id (o): ZIP4x4-ZIP4_2-1.2.14 0 0 IN IP4 10.1.71.152 Session Name (s): zultys media Connection Information (c): IN IP4 10.1.71.152 Time Description, active time (t): 0 0 Media Description, name and address (m): audio 33000 RTP/AVP 0 8 18 101 Media Attribute (a): rtpmap:0 PCMU/8000 Media Attribute (a): rtpmap:8 PCMA/8000 Media Attribute (a): rtpmap:18 G729/8000 Media Attribute (a): rtpmap:101 telephone-event/8000 Media Attribute (a): fmtp:101 0-15 Media Attribute (a): ptime:20 |
| 4. | Internet Protocol, Src Addr: 10.1.71.150 (10.1.71.150), Dst Addr: SIP (10.1.40.25) User Datagram Protocol, Src Port: 5060 (5060), Dst Port: 5060 (5060) Session Initiation Protocol Status line: SIP/2.0 100 Trying Message Header From: "ZIP4_2" <sip:ZIP4_2@10.1.40.25:5060>;tag=zip4x4_3571429956-104 To: "ZIP4_1" <sip:ZIP4_1@10.1.40.25:5060>;tag=zip4x4_1121346797-183 Call-ID: 3571429538-104 CSeq: 1 INVITE Via: SIP/2.0/UDP 10.1.40.25:5060;branch=1_1TaoTXPDdJUNvwg6LKVRfQ99,SIP/2.0/UDP 10.1.71.152:5060 Contact: ZIP4_1 <sip:ZIP4_1@10.1.71.150:5060> Content-Length: 0 |
| 5. | Internet Protocol, Src Addr: 10.1.71.150 (10.1.71.150), Dst Addr: SIP (10.1.40.25) User Datagram Protocol, Src Port: 5060 (5060), Dst Port: 5060 (5060) Session Initiation Protocol Status line: SIP/2.0 180 Ringing Message Header From: "ZIP4_2" <sip:ZIP4_2@10.1.40.25:5060>;tag=zip4x4_3571429956-104 To: "ZIP4_1" <sip:ZIP4_1@10.1.40.25:5060>;tag=zip4x4_1121346797-183 Call-ID: 3571429538-104 CSeq: 1 INVITE Via: SIP/2.0/UDP 10.1.40.25:5060;branch=1_1TaoTXPDdJUNvwg6LKVRfQ99,SIP/2.0/UDP 10.1.71.152:5060 Contact: ZIP4_1 <sip:ZIP4_1@10.1.71.150:5060> Content-Length: 0 |
| 6. | Internet Protocol, Src Addr: SIP (10.1.40.25), Dst Addr: 10.1.71.152 (10.1.71.152) User Datagram Protocol, Src Port: 4283 (4283), Dst Port: 5060 (5060) Session Initiation Protocol Status line: SIP/2.0 180 Ringing Message Header From: "ZIP4_2" <sip:ZIP4_2@10.1.40.25:5060>;tag=zip4x4_3571429956-104 To: "ZIP4_1" <sip:ZIP4_1@10.1.40.25:5060>;tag=zip4x4_1121346797-183 Call-ID: 3571429538-104 CSeq: 1 INVITE Via: SIP/2.0/UDP 10.1.71.152:5060 Contact:ZIP4_1 <sip:ZIP4_1@10.1.71.150:5060> Content-Length: 0 |
| 7. | Internet Protocol, Src Addr: 10.1.71.150 (10.1.71.150), Dst Addr: SIP (10.1.40.25) User Datagram Protocol, Src Port: 5060 (5060), Dst Port: 5060 (5060) Session Initiation Protocol Status line: SIP/2.0 200 OK Message Header From: "ZIP4_2" <sip:ZIP4_2@10.1.40.25:5060>;tag=zip4x4_3571429956-104 To: "ZIP4_1" <sip:ZIP4_1@10.1.40.25:5060>;tag=zip4x4_1121346797-183 Call-ID: 3571429538-104 CSeq: 1 INVITE Via: SIP/2.0/UDP 10.1.40.25:5060;branch=1_1TaoTXPDdJUNvwg6LKVRfQ99,SIP/2.0/UDP 10.1.71.152:5060 Contact: ZIP4_1 <sip:ZIP4_1@10.1.71.150:5060> Allow: INVITE,BYE,CANCEL,ACK,OPTIONS,REGISTER,NOTIFY,MESSAGE,REFER Content-Type: application/sdp Content-Length: 212 Session Description Protocol Session Description Protocol Version (v): 0 Owner/Creator, Session Id (o): ZIP4x4-ZIP4_1-1.2.11 0 0 IN IP4 10.1.71.150 Session Name (s): zultys media Connection Information (c): IN IP4 10.1.71.150 Time Description, active time (t): 0 0 Media Description, name and address (m): audio 19996 RTP/AVP 0 101 Media Attribute (a): rtpmap:0 PCMU/8000 Media Attribute (a): ptime:20 Media Attribute (a): rtpmap:101 telephone-event/8000 Media Attribute (a): fmtp:101 0-15 |
| 8. | Internet Protocol, Src Addr: SIP (10.1.40.25), Dst Addr: 10.1.71.152 (10.1.71.152) User Datagram Protocol, Src Port: 4283 (4283), Dst Port: 5060 (5060) Session Initiation Protocol Status line: SIP/2.0 200 OK Message Header From: "ZIP4_2" <sip:ZIP4_2@10.1.40.25:5060>;tag=zip4x4_3571429956-104 To: "ZIP4_1" <sip:ZIP4_1@10.1.40.25:5060>;tag=zip4x4_1121346797-183 Call-ID: 3571429538-104 CSeq: 1 INVITE Via: SIP/2.0/UDP 10.1.71.152:5060 Contact:ZIP4_1 <sip:ZIP4_1@10.1.71.150:5060> Allow:INVITE,BYE,CANCEL,ACK,OPTIONS,REGISTER,NOTIFY,MESSAGE,REFER Content-Type:application/sdp Content-Length: 212 Session Description Protocol Session Description Protocol Version (v): 0 Owner/Creator, Session Id (o): ZIP4x4-ZIP4_1-1.2.11 0 0 IN IP4 10.1.71.150 Session Name (s): zultys media Connection Information (c): IN IP4 10.1.71.150 Time Description, active time (t): 0 0 Media Description, name and address (m): audio 19996 RTP/AVP 0 101 Media Attribute (a): rtpmap:0 PCMU/8000 Media Attribute (a): ptime:20 Media Attribute (a): rtpmap:101 telephone-event/8000 Media Attribute (a): fmtp:101 0-15 |
| 9. | Internet Protocol, Src Addr: 10.1.71.152 (10.1.71.152), Dst Addr: SIP (10.1.40.25) User Datagram Protocol, Src Port: 5060 (5060), Dst Port: 5060 (5060) Session Initiation Protocol Request line: ACK sip:ZIP4_1@10.1.40.25:5060 SIP/2.0 Message Header From: "ZIP4_2" <sip:ZIP4_2@10.1.40.25:5060>;tag=zip4x4_3571429956-104 To: "ZIP4_1" <sip:ZIP4_1@10.1.40.25:5060>;tag=zip4x4_1121346797-183 Call-ID: 3571429538-104 CSeq: 1 ACK Via: SIP/2.0/UDP 10.1.71.152:5060 Contact: ZIP4_2 <sip:ZIP4_2@10.1.71.152:5060> Content-Length: 0 |
| 10. | Internet Protocol, Src Addr: SIP (10.1.40.25), Dst Addr: 10.1.71.150 (10.1.71.150) User Datagram Protocol, Src Port: 4284 (4284), Dst Port: 5060 (5060) Session Initiation Protocol Request line: ACK sip:ZIP4_1@10.1.71.150:5060 SIP/2.0 Message Header From: "ZIP4_2" <sip:ZIP4_2@10.1.40.25:5060>;tag=zip4x4_3571429956-104 To: "ZIP4_1" <sip:ZIP4_1@10.1.40.25:5060>;tag=zip4x4_1121346797-183 Call-ID: 3571429538-104 CSeq: 1 ACK Via: SIP/2.0/UDP 10.1.40.25:5060;branch=1_1TaoTXPDdJUNvwg6LKVRfQ99 Via: SIP/2.0/UDP 10.1.71.152:5060 Contact:ZIP4_2 <sip:ZIP4_2@10.1.71.152:5060> Content-Length: 0 |
| 11. | Internet Protocol, Src Addr: 10.1.71.152 (10.1.71.152), Dst Addr: SIP (10.1.40.25) User Datagram Protocol, Src Port: 5060 (5060), Dst Port: 5060 (5060) Session Initiation Protocol Request line: INVITE sip:ZIP4_1@10.1.40.25:5060 SIP/2.0 Message Header From: "ZIP4_2" <sip:ZIP4_2@10.1.40.25:5060>;tag=zip4x4_3571429956-104 To: "ZIP4_1" <sip:ZIP4_1@10.1.40.25:5060>;tag=zip4x4_1121346797-183 Call-ID: 3571429538-104 CSeq: 2 INVITE Via: SIP/2.0/UDP 10.1.71.152:5060 Contact: ZIP4_2 <sip:ZIP4_2@10.1.71.152:5060> Content-Type: application/sdp Content-Length: 207 Session Description Protocol Session Description Protocol Version (v): 0 Owner/Creator, Session Id (o): ZIP4x4-ZIP4_2-1.2.14 0 1 IN IP4 10.1.71.152 Session Name (s): zultys media Connection Information (c): IN IP4 0.0.0.0 Time Description, active time (t): 0 0 Media Description, name and address (m): audio 1962 RTP/AVP 0 101 Media Attribute (a): rtpmap:0 PCMU/8000 Media Attribute (a): ptime:20 Media Attribute (a): rtpmap:101 telephone-event/8000 Media Attribute (a): fmtp:101 0-15 |
| 12. | Internet Protocol, Src Addr: SIP (10.1.40.25), Dst Addr: 10.1.71.152 (10.1.71.152) User Datagram Protocol, Src Port: 4283 (4283), Dst Port: 5060 (5060) Session Initiation Protocol Status line: SIP/2.0 100 Trying Message Header Via: SIP/2.0/UDP 10.1.71.152:5060 From: "ZIP4_2" <sip:ZIP4_2@10.1.40.25:5060>;tag=zip4x4_3571429956-104 To: "ZIP4_1" <sip:ZIP4_1@10.1.40.25:5060>;tag=zip4x4_1121346797-183 Call-ID: 3571429538-104 CSeq: 2 INVITE Content-Length: 0 |
| 13. | Internet Protocol, Src Addr: SIP (10.1.40.25), Dst Addr: 10.1.71.150 (10.1.71.150) User Datagram Protocol, Src Port: 4284 (4284), Dst Port: 5060 (5060) Session Initiation Protocol Request line: INVITE sip:ZIP4_1@10.1.71.150 SIP/2.0 Message Header From: "ZIP4_2" <sip:ZIP4_2@10.1.40.25:5060>;tag=zip4x4_3571429956-104 To: "ZIP4_1" <sip:ZIP4_1@10.1.40.25:5060>;tag=zip4x4_1121346797-183 Call-ID: 3571429538-104 CSeq: 2 INVITE Via: SIP/2.0/UDP 10.1.40.25:5060;branch=1_1TaoTXPDdJUNvwg6LKVRfQ99 Via: SIP/2.0/UDP 10.1.71.152:5060 Contact:ZIP4_2 <sip:ZIP4_2@10.1.71.152:5060> Content-Type:application/sdp Content-Length: 207 Session Description Protocol Session Description Protocol Version (v): 0 Owner/Creator, Session Id (o): ZIP4x4-ZIP4_2-1.2.14 0 1 IN IP4 10.1.71.152 Session Name (s): zultys media Connection Information (c): IN IP4 0.0.0.0 Time Description, active time (t): 0 0 Media Description, name and address (m): audio 1962 RTP/AVP 0 101 Media Attribute (a): rtpmap:0 PCMU/8000 Media Attribute (a): ptime:20 Media Attribute (a): rtpmap:101 telephone-event/8000 Media Attribute (a): fmtp:101 0-15 |
| 14. | Internet Protocol, Src Addr: 10.1.71.150 (10.1.71.150), Dst Addr: SIP (10.1.40.25) User Datagram Protocol, Src Port: 5060 (5060), Dst Port: 5060 (5060) Session Initiation Protocol Status line: SIP/2.0 200 OK Message Header From: "ZIP4_2" <sip:ZIP4_2@10.1.40.25:5060>;tag=zip4x4_3571429956-104 To: "ZIP4_1" <sip:ZIP4_1@10.1.40.25:5060>;tag=zip4x4_1121346797-183 Call-ID: 3571429538-104 CSeq: 2 INVITE Via: SIP/2.0/UDP 10.1.40.25:5060;branch=1_1TaoTXPDdJUNvwg6LKVRfQ99,SIP/2.0/UDP 10.1.71.152:5060 Contact: ZIP4_1 <sip:ZIP4_1@10.1.71.150:5060> Allow: INVITE,BYE,CANCEL,ACK,OPTIONS,REGISTER,NOTIFY,MESSAGE,REFER Content-Type: application/sdp Content-Length: 208 Session Description Protocol Session Description Protocol Version (v): 0 Owner/Creator, Session Id (o): ZIP4x4-ZIP4_1-1.2.11 0 0 IN IP4 10.1.71.150 Session Name (s): zultys media Connection Information (c): IN IP4 0.0.0.0 Time Description, active time (t): 0 0 Media Description, name and address (m): audio 19996 RTP/AVP 0 101 Media Attribute (a): rtpmap:0 PCMU/8000 Media Attribute (a): ptime:20 Media Attribute (a): rtpmap:101 telephone-event/8000 Media Attribute (a): fmtp:101 0-15 |
| 15. | Internet Protocol, Src Addr: SIP (10.1.40.25), Dst Addr: 10.1.71.152 (10.1.71.152) User Datagram Protocol, Src Port: 4283 (4283), Dst Port: 5060 (5060) Session Initiation Protocol Status line: SIP/2.0 200 OK Message Header From: "ZIP4_2" <sip:ZIP4_2@10.1.40.25:5060>;tag=zip4x4_3571429956-104 To: "ZIP4_1" <sip:ZIP4_1@10.1.40.25:5060>;tag=zip4x4_1121346797-183 Call-ID: 3571429538-104 CSeq: 2 INVITE Via: SIP/2.0/UDP 10.1.71.152:5060 Contact:ZIP4_1 <sip:ZIP4_1@10.1.71.150:5060> Allow:INVITE,BYE,CANCEL,ACK,OPTIONS,REGISTER,NOTIFY,MESSAGE,REFER Content-Type:application/sdp Content-Length: 208 Session Description Protocol Session Description Protocol Version (v): 0 Owner/Creator, Session Id (o): ZIP4x4-ZIP4_1-1.2.11 0 0 IN IP4 10.1.71.150 Session Name (s): zultys media Connection Information (c): IN IP4 0.0.0.0 Time Description, active time (t): 0 0 Media Description, name and address (m): audio 19996 RTP/AVP 0 101 Media Attribute (a): rtpmap:0 PCMU/8000 Media Attribute (a): ptime:20 Media Attribute (a): rtpmap:101 telephone-event/8000 Media Attribute (a): fmtp:101 0-15 |
| 16. | Internet Protocol, Src Addr: 10.1.71.152 (10.1.71.152), Dst Addr: SIP (10.1.40.25) User Datagram Protocol, Src Port: 5060 (5060), Dst Port: 5060 (5060) Session Initiation Protocol Request line: ACK sip:ZIP4_1@10.1.40.25:5060 SIP/2.0 Message Header From: "ZIP4_2" <sip:ZIP4_2@10.1.40.25:5060>;tag=zip4x4_3571429956-104 To: "ZIP4_1" <sip:ZIP4_1@10.1.40.25:5060>;tag=zip4x4_1121346797-183 Call-ID: 3571429538-104 CSeq: 2 ACK Via: SIP/2.0/UDP 10.1.71.152:5060 Contact: ZIP4_2 <sip:ZIP4_2@10.1.71.152:5060> Content-Length: 0 |
| 17. | Internet Protocol, Src Addr: SIP (10.1.40.25), Dst Addr: 10.1.71.150 (10.1.71.150) User Datagram Protocol, Src Port: 4284 (4284), Dst Port: 5060 (5060) Session Initiation Protocol Request line: ACK sip:ZIP4_1@10.1.71.150:5060 SIP/2.0 Message Header From: "ZIP4_2" <sip:ZIP4_2@10.1.40.25:5060>;tag=zip4x4_3571429956-104 To: "ZIP4_1" <sip:ZIP4_1@10.1.40.25:5060>;tag=zip4x4_1121346797-183 Call-ID: 3571429538-104 CSeq: 2 ACK Via: SIP/2.0/UDP 10.1.40.25:5060;branch=1_1TaoTXPDdJUNvwg6LKVRfQ99 Via: SIP/2.0/UDP 10.1.71.152:5060 Contact:ZIP4_2 <sip:ZIP4_2@10.1.71.152:5060> Content-Length: 0 |
| 18. | Internet Protocol, Src Addr: 10.1.71.152 (10.1.71.152), Dst Addr: SIP (10.1.40.25) User Datagram Protocol, Src Port: 5060 (5060), Dst Port: 5060 (5060) Session Initiation Protocol Request line: INVITE sip:ZIP4_1@10.1.40.25:5060 SIP/2.0 Message Header From: "ZIP4_2" <sip:ZIP4_2@10.1.40.25:5060>;tag=zip4x4_3571429956-104 To: "ZIP4_1" <sip:ZIP4_1@10.1.40.25:5060>;tag=zip4x4_1121346797-183 Call-ID: 3571429538-104 CSeq: 3 INVITE Via: SIP/2.0/UDP 10.1.71.152:5060 Contact: ZIP4_2 <sip:ZIP4_2@10.1.71.152:5060> Allow: INVITE,BYE,CANCEL,ACK,OPTIONS,REGISTER,NOTIFY,MESSAGE,REFER User-Agent: Zultys ZIP4x4 1.2.14 Content-Type: application/sdp Content-Length: 262 Session Description Protocol Session Description Protocol Version (v): 0 Owner/Creator, Session Id (o): ZIP4x4-ZIP4_2-1.2.14 0 0 IN IP4 10.1.71.152 Session Name (s): zultys media Connection Information (c): IN IP4 10.1.71.152 Time Description, active time (t): 0 0 Media Description, name and address (m): audio 33000 RTP/AVP 0 8 18 101 Media Attribute (a): rtpmap:0 PCMU/8000 Media Attribute (a): rtpmap:8 PCMA/8000 Media Attribute (a): rtpmap:18 G729/8000 Media Attribute (a): rtpmap:101 telephone-event/8000 Media Attribute (a): fmtp:101 0-15 Media Attribute (a): ptime:20 |
| 19. | Internet Protocol, Src Addr: SIP (10.1.40.25), Dst Addr: 10.1.71.152 (10.1.71.152) User Datagram Protocol, Src Port: 4283 (4283), Dst Port: 5060 (5060) Session Initiation Protocol Status line: SIP/2.0 100 Trying Message Header Via: SIP/2.0/UDP 10.1.71.152:5060 From: "ZIP4_2" <sip:ZIP4_2@10.1.40.25:5060>;tag=zip4x4_3571429956-104 To: "ZIP4_1" <sip:ZIP4_1@10.1.40.25:5060>;tag=zip4x4_1121346797-183 Call-ID: 3571429538-104 CSeq: 3 INVITE Content-Length: 0 |
| 20. | Internet Protocol, Src Addr: SIP (10.1.40.25), Dst Addr: 10.1.71.150 (10.1.71.150) User Datagram Protocol, Src Port: 4284 (4284), Dst Port: 5060 (5060) Session Initiation Protocol Request line: INVITE sip:ZIP4_1@10.1.71.150 SIP/2.0 Message Header From: "ZIP4_2" <sip:ZIP4_2@10.1.40.25:5060>;tag=zip4x4_3571429956-104 To: "ZIP4_1" <sip:ZIP4_1@10.1.40.25:5060>;tag=zip4x4_1121346797-183 Call-ID: 3571429538-104 CSeq: 3 INVITE Via: SIP/2.0/UDP 10.1.40.25:5060;branch=1_1TaoTXPDdJUNvwg6LKVRfQ99 Via: SIP/2.0/UDP 10.1.71.152:5060 Contact:ZIP4_2 <sip:ZIP4_2@10.1.71.152:5060> Allow:INVITE,BYE,CANCEL,ACK,OPTIONS,REGISTER,NOTIFY,MESSAGE,REFER User-Agent:Zultys ZIP4x4 1.2.14 Content-Type:application/sdp Content-Length: 262 Session Description Protocol Session Description Protocol Version (v): 0 Owner/Creator, Session Id (o): ZIP4x4-ZIP4_2-1.2.14 0 0 IN IP4 10.1.71.152 Session Name (s): zultys media Connection Information (c): IN IP4 10.1.71.152 Time Description, active time (t): 0 0 Media Description, name and address (m): audio 33000 RTP/AVP 0 8 18 101 Media Attribute (a): rtpmap:0 PCMU/8000 Media Attribute (a): rtpmap:8 PCMA/8000 Media Attribute (a): rtpmap:18 G729/8000 Media Attribute (a): rtpmap:101 telephone-event/8000 Media Attribute (a): fmtp:101 0-15 Media Attribute (a): ptime:20 |
| 21. | Internet Protocol, Src Addr: 10.1.71.150 (10.1.71.150), Dst Addr: SIP (10.1.40.25) User Datagram Protocol, Src Port: 5060 (5060), Dst Port: 5060 (5060) Session Initiation Protocol Status line: SIP/2.0 200 OK Message Header From: "ZIP4_2" <sip:ZIP4_2@10.1.40.25:5060>;tag=zip4x4_3571429956-104 To: "ZIP4_1" <sip:ZIP4_1@10.1.40.25:5060>;tag=zip4x4_1121346797-183 Call-ID: 3571429538-104 CSeq: 3 INVITE Via: SIP/2.0/UDP 10.1.40.25:5060;branch=1_1TaoTXPDdJUNvwg6LKVRfQ99,SIP/2.0/UDP 10.1.71.152:5060 Contact: ZIP4_1 <sip:ZIP4_1@10.1.71.150:5060> Allow: INVITE,BYE,CANCEL,ACK,OPTIONS,REGISTER,NOTIFY,MESSAGE,REFER Content-Type: application/sdp Content-Length: 212 Session Description Protocol Session Description Protocol Version (v): 0 Owner/Creator, Session Id (o): ZIP4x4-ZIP4_1-1.2.11 0 0 IN IP4 10.1.71.150 Session Name (s): zultys media Connection Information (c): IN IP4 10.1.71.150 Time Description, active time (t): 0 0 Media Description, name and address (m): audio 19996 RTP/AVP 0 101 Media Attribute (a): rtpmap:0 PCMU/8000 Media Attribute (a): ptime:20 Media Attribute (a): rtpmap:101 telephone-event/8000 Media Attribute (a): fmtp:101 0-15 |
| 22. | Internet Protocol, Src Addr: SIP (10.1.40.25), Dst Addr: 10.1.71.152 (10.1.71.152) User Datagram Protocol, Src Port: 4283 (4283), Dst Port: 5060 (5060) Session Initiation Protocol Status line: SIP/2.0 200 OK Message Header From: "ZIP4_2" <sip:ZIP4_2@10.1.40.25:5060>;tag=zip4x4_3571429956-104 To: "ZIP4_1" <sip:ZIP4_1@10.1.40.25:5060>;tag=zip4x4_1121346797-183 Call-ID: 3571429538-104 CSeq: 3 INVITE Via: SIP/2.0/UDP 10.1.71.152:5060 Contact:ZIP4_1 <sip:ZIP4_1@10.1.71.150:5060> Allow:INVITE,BYE,CANCEL,ACK,OPTIONS,REGISTER,NOTIFY,MESSAGE,REFER Content-Type:application/sdp Content-Length: 212 Session Description Protocol Session Description Protocol Version (v): 0 Owner/Creator, Session Id (o): ZIP4x4-ZIP4_1-1.2.11 0 0 IN IP4 10.1.71.150 Session Name (s): zultys media Connection Information (c): IN IP4 10.1.71.150 Time Description, active time (t): 0 0 Media Description, name and address (m): audio 19996 RTP/AVP 0 101 Media Attribute (a): rtpmap:0 PCMU/8000 Media Attribute (a): ptime:20 Media Attribute (a): rtpmap:101 telephone-event/8000 Media Attribute (a): fmtp:101 0-15 |
| 23. | Internet Protocol, Src Addr: 10.1.71.152 (10.1.71.152), Dst Addr: SIP (10.1.40.25) User Datagram Protocol, Src Port: 5060 (5060), Dst Port: 5060 (5060) Session Initiation Protocol Request line: ACK sip:ZIP4_1@10.1.40.25:5060 SIP/2.0 Message Header From: "ZIP4_2" <sip:ZIP4_2@10.1.40.25:5060>;tag=zip4x4_3571429956-104 To: "ZIP4_1" <sip:ZIP4_1@10.1.40.25:5060>;tag=zip4x4_1121346797-183 Call-ID: 3571429538-104 CSeq: 3 ACK Via: SIP/2.0/UDP 10.1.71.152:5060 Contact: ZIP4_2 <sip:ZIP4_2@10.1.71.152:5060> Content-Length: 0 |
| 24. | Internet Protocol, Src Addr: SIP (10.1.40.25), Dst Addr: 10.1.71.150 (10.1.71.150) User Datagram Protocol, Src Port: 4284 (4284), Dst Port: 5060 (5060) Session Initiation Protocol Request line: ACK sip:ZIP4_1@10.1.71.150:5060 SIP/2.0 Message Header From: "ZIP4_2" <sip:ZIP4_2@10.1.40.25:5060>;tag=zip4x4_3571429956-104 To: "ZIP4_1" <sip:ZIP4_1@10.1.40.25:5060>;tag=zip4x4_1121346797-183 Call-ID: 3571429538-104 CSeq: 3 ACK Via: SIP/2.0/UDP 10.1.40.25:5060;branch=1_1TaoTXPDdJUNvwg6LKVRfQ99 Via: SIP/2.0/UDP 10.1.71.152:5060 Contact:ZIP4_2 <sip:ZIP4_2@10.1.71.152:5060> Content-Length: 0 |
| 25. | Internet Protocol, Src Addr: 10.1.71.152 (10.1.71.152), Dst Addr: SIP (10.1.40.25) User Datagram Protocol, Src Port: 5060 (5060), Dst Port: 5060 (5060) Session Initiation Protocol Request line: BYE sip:ZIP4_1@10.1.40.25:5060 SIP/2.0 Message Header From: "ZIP4_2" <sip:ZIP4_2@10.1.40.25:5060>;tag=zip4x4_3571429956-104 To: "ZIP4_1" <sip:ZIP4_1@10.1.40.25:5060>;tag=zip4x4_1121346797-183 Call-ID: 3571429538-104 CSeq: 4 BYE Via: SIP/2.0/UDP 10.1.71.152:5060 Content-Length: 0 |
| 26. | Internet Protocol, Src Addr: SIP (10.1.40.25), Dst Addr: 10.1.71.150 (10.1.71.150) User Datagram Protocol, Src Port: 4284 (4284), Dst Port: 5060 (5060) Session Initiation Protocol Request line: BYE sip:ZIP4_1@10.1.71.150 SIP/2.0 Message Header From: "ZIP4_2" <sip:ZIP4_2@10.1.40.25:5060>;tag=zip4x4_3571429956-104 To: "ZIP4_1" <sip:ZIP4_1@10.1.40.25:5060>;tag=zip4x4_1121346797-183 Call-ID: 3571429538-104 CSeq: 4 BYE Via: SIP/2.0/UDP 10.1.40.25:5060;branch=1_1TaoTXPDdJUNvwg6LKVRfQ99 Via: SIP/2.0/UDP 10.1.71.152:5060 Content-Length: 0 |
| 27. | Internet Protocol, Src Addr: 10.1.71.150 (10.1.71.150), Dst Addr: SIP (10.1.40.25) User Datagram Protocol, Src Port: 5060 (5060), Dst Port: 5060 (5060) Session Initiation Protocol Status line: SIP/2.0 200 OK Message Header From: "ZIP4_2" <sip:ZIP4_2@10.1.40.25:5060>;tag=zip4x4_3571429956-104 To: "ZIP4_1" <sip:ZIP4_1@10.1.40.25:5060>;tag=zip4x4_1121346797-183 Call-ID: 3571429538-104 CSeq: 4 BYE Via: SIP/2.0/UDP 10.1.40.25:5060;branch=1_1TaoTXPDdJUNvwg6LKVRfQ99,SIP/2.0/UDP 10.1.71.152:5060 Contact: ZIP4_1 <sip:ZIP4_1@10.1.71.150:5060> Allow: INVITE,BYE,CANCEL,ACK,OPTIONS,REGISTER,NOTIFY,MESSAGE,REFER Content-Length: 0 |
| 28. | Internet Protocol, Src Addr: SIP (10.1.40.25), Dst Addr: 10.1.71.152 (10.1.71.152) User Datagram Protocol, Src Port: 4283 (4283), Dst Port: 5060 (5060) Session Initiation Protocol Status line: SIP/2.0 200 OK Message Header From: "ZIP4_2" <sip:ZIP4_2@10.1.40.25:5060>;tag=zip4x4_3571429956-104 To: "ZIP4_1" <sip:ZIP4_1@10.1.40.25:5060>;tag=zip4x4_1121346797-183 Call-ID: 3571429538-104 CSeq: 4 BYE Via: SIP/2.0/UDP 10.1.71.152:5060 Contact:ZIP4_1 <sip:ZIP4_1@10.1.71.150:5060> Allow:INVITE,BYE,CANCEL,ACK,OPTIONS,REGISTER,NOTIFY,MESSAGE,REFER Content-Length: 0 |

# Blind Transfer

A blind transfer is one in which the person transferring the call does not know the outcome of the transfer. With a blind transfer the call between the first two parties is temporarily placed on hold and is then cleared once the transfer is completes. In step 11 the phone indicates in the SDP that the connection information is IN IP4 0.0.0.0. This stops media from being sent to the previous address. This is one approach to placing a call on hold. Another approach is to use a=sendonly or a=inactive within the SDP.

In step 18 the REFER method is used to initiate the call transfer. An older way of initiating a call transfer was to add an Also header in the BYE (e.g., Also:<sip:6666@10.1.10.100>). After receiving this, a UA would initiate an outgoing call to the specified URI.

Figure 6: Blind Transfer

| 1. | Internet Protocol, Src Addr: 10.1.71.172 (10.1.71.172), Dst Addr: 10.1.70.249 (10.1.70.249) User Datagram Protocol, Src Port: 5060 (5060), Dst Port: 5060 (5060) Session Initiation Protocol Request line: INVITE sip:pat@10.1.70.249:5060 SIP/2.0 Message Header From: "ray" <sip:ray@zultys.com:5060>;tag=zip4x4_4000276508-43 To: "pat" <sip:pat@10.1.70.249:5060> Call-ID: 4000276057-43 CSeq: 1 INVITE Via: SIP/2.0/UDP 10.1.71.172:5060 Contact: ray <sip:ray@10.1.71.172:5060> Allow: INVITE,BYE,CANCEL,ACK,OPTIONS,REGISTER,NOTIFY,MESSAGE,REFER User-Agent: Zultys ZIP4x4 1.2.11 Content-Type: application/sdp Content-Length: 259 Session Description Protocol Session Description Protocol Version (v): 0 Owner/Creator, Session Id (o): ZIP4x4-ray-1.2.11 0 0 IN IP4 10.1.71.172 Session Name (s): zultys media Connection Information (c): IN IP4 10.1.71.172 Time Description, active time (t): 0 0 Media Description, name and address (m): audio 19996 RTP/AVP 0 8 18 101 Media Attribute (a): rtpmap:0 PCMU/8000 Media Attribute (a): rtpmap:8 PCMA/8000 Media Attribute (a): rtpmap:18 G729/8000 Media Attribute (a): rtpmap:101 telephone-event/8000 Media Attribute (a): fmtp:101 0-15 Media Attribute (a): ptime:20 |
| --- | --- |
| 2. | Internet Protocol, Src Addr: 10.1.70.249 (10.1.70.249), Dst Addr: 10.1.71.172 (10.1.71.172) User Datagram Protocol, Src Port: 5060 (5060), Dst Port: 5060 (5060) Session Initiation Protocol Status line: SIP/2.0 100 trying -- your call is important to us Message Header From: "ray" <sip:ray@zultys.com:5060>;tag=zip4x4_4000276508-43 To: "pat" <sip:pat@10.1.70.249:5060> Call-ID: 4000276057-43 CSeq: 1 INVITE Via: SIP/2.0/UDP 10.1.71.172:5060 Server: Sip EXpress router (0.8.12 (i386/linux)) Content-Length: 0 Warning: 392 10.1.70.249:5060 "Noisy feedback tells: pid=7848 req_src_ip=10.1.71.172 req_src_port=5060 in_uri=sip:pat@10.1.70.249:5060 out_uri=sip:pat@10.1.71.187:5060 via_cnt==1" |
| 3. | Internet Protocol, Src Addr: 10.1.70.249 (10.1.70.249), Dst Addr: 10.1.71.187 (10.1.71.187) User Datagram Protocol, Src Port: 5060 (5060), Dst Port: 5060 (5060) Session Initiation Protocol Request line: INVITE sip:pat@10.1.71.187:5060 SIP/2.0 Message Header Max-Forwards: 10 Record-Route: <sip:pat@10.1.70.249;ftag=zip4x4_4000276508-43;lr=on> From: "ray" <sip:ray@zultys.com:5060>;tag=zip4x4_4000276508-43 To: "pat" <sip:pat@10.1.70.249:5060> Call-ID: 4000276057-43 CSeq: 1 INVITE Via: SIP/2.0/UDP 10.1.70.249;branch=z9hG4bK76ef.f2f10a64.0 Via: SIP/2.0/UDP 10.1.71.172:5060 Contact: ray <sip:ray@10.1.71.172:5060> Allow: INVITE,BYE,CANCEL,ACK,OPTIONS,REGISTER,NOTIFY,MESSAGE,REFER User-Agent: Zultys ZIP4x4 1.2.11 Content-Type: application/sdp Content-Length: 259 P-hint: USRLOC Session Description Protocol Session Description Protocol Version (v): 0 Owner/Creator, Session Id (o): ZIP4x4-ray-1.2.11 0 0 IN IP4 10.1.71.172 Session Name (s): zultys media Connection Information (c): IN IP4 10.1.71.172 Time Description, active time (t): 0 0 Media Description, name and address (m): audio 19996 RTP/AVP 0 8 18 101 Media Attribute (a): rtpmap:0 PCMU/8000 Media Attribute (a): rtpmap:8 PCMA/8000 Media Attribute (a): rtpmap:18 G729/8000 Media Attribute (a): rtpmap:101 telephone-event/8000 Media Attribute (a): fmtp:101 0-15 Media Attribute (a): ptime:20 |
| 4. | Internet Protocol, Src Addr: 10.1.71.187 (10.1.71.187), Dst Addr: 10.1.70.249 (10.1.70.249) User Datagram Protocol, Src Port: 5060 (5060), Dst Port: 5060 (5060) Session Initiation Protocol Status line: SIP/2.0 100 Trying Message Header From: "ray" <sip:ray@zultys.com:5060>;tag=zip4x4_4000276508-43 To: "pat" <sip:pat@10.1.70.249:5060>;tag=zip4x4_3110815550-42 Call-ID: 4000276057-43 CSeq: 1 INVITE Via: SIP/2.0/UDP 10.1.70.249;branch=z9hG4bK76ef.f2f10a64.0,SIP/2.0/UDP 10.1.71.172:5060 Record-Route: <sip:pat@10.1.70.249;ftag=zip4x4_4000276508-43;lr=on> Contact: pat <sip:pat@10.1.71.187:5060> Content-Length: 0 |
| 5. | Internet Protocol, Src Addr: 10.1.71.187 (10.1.71.187), Dst Addr: 10.1.70.249 (10.1.70.249) User Datagram Protocol, Src Port: 5060 (5060), Dst Port: 5060 (5060) Session Initiation Protocol Status line: SIP/2.0 180 Ringing Message Header From: "ray" <sip:ray@zultys.com:5060>;tag=zip4x4_4000276508-43 To: "pat" <sip:pat@10.1.70.249:5060>;tag=zip4x4_3110815550-42 Call-ID: 4000276057-43 CSeq: 1 INVITE Via: SIP/2.0/UDP 10.1.70.249;branch=z9hG4bK76ef.f2f10a64.0,SIP/2.0/UDP 10.1.71.172:5060 Record-Route: <sip:pat@10.1.70.249;ftag=zip4x4_4000276508-43;lr=on> Contact: pat <sip:pat@10.1.71.187:5060> Content-Length: 0 |
| 6. | Internet Protocol, Src Addr: 10.1.70.249 (10.1.70.249), Dst Addr: 10.1.71.172 (10.1.71.172) User Datagram Protocol, Src Port: 5060 (5060), Dst Port: 5060 (5060) Session Initiation Protocol Status line: SIP/2.0 180 Ringing Message Header From: "ray" <sip:ray@zultys.com:5060>;tag=zip4x4_4000276508-43 To: "pat" <sip:pat@10.1.70.249:5060>;tag=zip4x4_3110815550-42 Call-ID: 4000276057-43 CSeq: 1 INVITE Via: SIP/2.0/UDP 10.1.71.172:5060 Record-Route: <sip:pat@10.1.70.249;ftag=zip4x4_4000276508-43;lr=on> Contact: pat <sip:pat@10.1.71.187:5060> Content-Length: 0 |
| 7. | Internet Protocol, Src Addr: 10.1.71.187 (10.1.71.187), Dst Addr: 10.1.70.249 (10.1.70.249) User Datagram Protocol, Src Port: 5060 (5060), Dst Port: 5060 (5060) Session Initiation Protocol Status line: SIP/2.0 200 OK Message Header From: "ray" <sip:ray@zultys.com:5060>;tag=zip4x4_4000276508-43 To: "pat" <sip:pat@10.1.70.249:5060>;tag=zip4x4_3110815550-42 Call-ID: 4000276057-43 CSeq: 1 INVITE Via: SIP/2.0/UDP 10.1.70.249;branch=z9hG4bK76ef.f2f10a64.0,SIP/2.0/UDP 10.1.71.172:5060 Record-Route: <sip:pat@10.1.70.249;ftag=zip4x4_4000276508-43;lr=on> Contact: pat <sip:pat@10.1.71.187:5060> Allow: INVITE,BYE,CANCEL,ACK,OPTIONS,REGISTER,NOTIFY,MESSAGE,REFER Content-Type: application/sdp Content-Length: 209 Session Description Protocol Session Description Protocol Version (v): 0 Owner/Creator, Session Id (o): ZIP4x4-pat-1.2.14 0 0 IN IP4 10.1.71.187 Session Name (s): zultys media Connection Information (c): IN IP4 10.1.71.187 Time Description, active time (t): 0 0 Media Description, name and address (m): audio 33000 RTP/AVP 0 101 Media Attribute (a): rtpmap:0 PCMU/8000 Media Attribute (a): ptime:20 Media Attribute (a): rtpmap:101 telephone-event/8000 Media Attribute (a): fmtp:101 0-15 |
| 8. | Internet Protocol, Src Addr: 10.1.70.249 (10.1.70.249), Dst Addr: 10.1.71.172 (10.1.71.172) User Datagram Protocol, Src Port: 5060 (5060), Dst Port: 5060 (5060) Session Initiation Protocol Status line: SIP/2.0 200 OK Message Header From: "ray" <sip:ray@zultys.com:5060>;tag=zip4x4_4000276508-43 To: "pat" <sip:pat@10.1.70.249:5060>;tag=zip4x4_3110815550-42 Call-ID: 4000276057-43 CSeq: 1 INVITE Via: SIP/2.0/UDP 10.1.71.172:5060 Record-Route: <sip:pat@10.1.70.249;ftag=zip4x4_4000276508-43;lr=on> Contact: pat <sip:pat@10.1.71.187:5060> Allow: INVITE,BYE,CANCEL,ACK,OPTIONS,REGISTER,NOTIFY,MESSAGE,REFER Content-Type: application/sdp Content-Length: 209 Session Description Protocol Session Description Protocol Version (v): 0 Owner/Creator, Session Id (o): ZIP4x4-pat-1.2.14 0 0 IN IP4 10.1.71.187 Session Name (s): zultys media Connection Information (c): IN IP4 10.1.71.187 Time Description, active time (t): 0 0 Media Description, name and address (m): audio 33000 RTP/AVP 0 101 Media Attribute (a): rtpmap:0 PCMU/8000 Media Attribute (a): ptime:20 Media Attribute (a): rtpmap:101 telephone-event/8000 Media Attribute (a): fmtp:101 0-15 |
| 9. | Internet Protocol, Src Addr: 10.1.71.172 (10.1.71.172), Dst Addr: 10.1.70.249 (10.1.70.249) User Datagram Protocol, Src Port: 5060 (5060), Dst Port: 5060 (5060) Session Initiation Protocol Request line: ACK sip:pat@10.1.70.249:5060 SIP/2.0 Message Header From: "ray" <sip:ray@zultys.com:5060>;tag=zip4x4_4000276508-43 To: "pat" <sip:pat@10.1.70.249:5060>;tag=zip4x4_3110815550-42 Call-ID: 4000276057-43 CSeq: 1 ACK Via: SIP/2.0/UDP 10.1.71.172:5060 Contact: ray <sip:ray@10.1.71.172:5060> Route: <sip:pat@10.1.70.249;ftag=zip4x4_4000276508-43;lr=on>,<sip:pat@10.1.71.187:5060> Content-Length: 0 |
| 10. | Internet Protocol, Src Addr: 10.1.70.249 (10.1.70.249), Dst Addr: 10.1.71.187 (10.1.71.187) User Datagram Protocol, Src Port: 5060 (5060), Dst Port: 5060 (5060) Session Initiation Protocol Request line: ACK sip:pat@10.1.71.187:5060 SIP/2.0 Message Header Max-Forwards: 10 Record-Route: <sip:pat@10.1.70.249;ftag=zip4x4_4000276508-43;lr=on> From: "ray" <sip:ray@zultys.com:5060>;tag=zip4x4_4000276508-43 To: "pat" <sip:pat@10.1.70.249:5060>;tag=zip4x4_3110815550-42 Call-ID: 4000276057-43 CSeq: 1 ACK Via: SIP/2.0/UDP 10.1.70.249;branch=0 Via: SIP/2.0/UDP 10.1.71.172:5060 Contact: ray <sip:ray@10.1.71.172:5060> Content-Length: 0 |
| 11. | Internet Protocol, Src Addr: 10.1.71.187 (10.1.71.187), Dst Addr: 10.1.70.249 (10.1.70.249) User Datagram Protocol, Src Port: 5060 (5060), Dst Port: 5060 (5060) Session Initiation Protocol Request line: INVITE sip:pat@10.1.70.249;ftag=zip4x4_4000276508-43;lr=on SIP/2.0 Message Header From: "pat" <sip:pat@10.1.70.249:5060>;tag=zip4x4_3110815550-42 To: "ray" <sip:ray@zultys.com:5060>;tag=zip4x4_4000276508-43 Call-ID: 4000276057-43 CSeq: 2 INVITE Via: SIP/2.0/UDP 10.1.71.187:5060 Contact: pat <sip:pat@10.1.71.187:5060> Route: <sip:ray@10.1.71.172:5060> Content-Type: application/sdp Content-Length: 205 Session Description Protocol Session Description Protocol Version (v): 0 Owner/Creator, Session Id (o): ZIP4x4-pat-1.2.14 0 1 IN IP4 10.1.71.187 Session Name (s): zultys media Connection Information (c): IN IP4 0.0.0.0 Time Description, active time (t): 0 0 Media Description, name and address (m): audio 33000 RTP/AVP 0 101 Media Attribute (a): rtpmap:0 PCMU/8000 Media Attribute (a): ptime:20 Media Attribute (a): rtpmap:101 telephone-event/8000 Media Attribute (a): fmtp:101 0-15 |
| 12. | Internet Protocol, Src Addr: 10.1.70.249 (10.1.70.249), Dst Addr: 10.1.71.187 (10.1.71.187) User Datagram Protocol, Src Port: 5060 (5060), Dst Port: 5060 (5060) Session Initiation Protocol Status line: SIP/2.0 100 trying -- your call is important to us Message Header From: "pat" <sip:pat@10.1.70.249:5060>;tag=zip4x4_3110815550-42 To: "ray" <sip:ray@zultys.com:5060>;tag=zip4x4_4000276508-43 Call-ID: 4000276057-43 CSeq: 2 INVITE Via: SIP/2.0/UDP 10.1.71.187:5060 Server: Sip EXpress router (0.8.12 (i386/linux)) Content-Length: 0 Warning: 392 10.1.70.249:5060 "Noisy feedback tells: pid=7848 req_src_ip=10.1.71.187 req_src_port=5060 in_uri=sip:pat@10.1.70.249;ftag=zip4x4_4000276508-43;lr=on out_uri=sip:ray@10.1.71.172:5060 via_cnt==1" |
| 13. | Internet Protocol, Src Addr: 10.1.70.249 (10.1.70.249), Dst Addr: 10.1.71.172 (10.1.71.172) User Datagram Protocol, Src Port: 5060 (5060), Dst Port: 5060 (5060) Session Initiation Protocol Request line: INVITE sip:ray@10.1.71.172:5060 SIP/2.0 Message Header Max-Forwards: 10 Record-Route: <sip:pat@10.1.70.249;ftag=zip4x4_3110815550-42;lr=on> From: "pat" <sip:pat@10.1.70.249:5060>;tag=zip4x4_3110815550-42 To: "ray" <sip:ray@zultys.com:5060>;tag=zip4x4_4000276508-43 Call-ID: 4000276057-43 CSeq: 2 INVITE Via: SIP/2.0/UDP 10.1.70.249;branch=z9hG4bK46ef.18da0543.0 Via: SIP/2.0/UDP 10.1.71.187:5060 Contact: pat <sip:pat@10.1.71.187:5060> Content-Type: application/sdp Content-Length: 205 Session Description Protocol Session Description Protocol Version (v): 0 Owner/Creator, Session Id (o): ZIP4x4-pat-1.2.14 0 1 IN IP4 10.1.71.187 Session Name (s): zultys media Connection Information (c): IN IP4 0.0.0.0 Time Description, active time (t): 0 0 Media Description, name and address (m): audio 33000 RTP/AVP 0 101 Media Attribute (a): rtpmap:0 PCMU/8000 Media Attribute (a): ptime:20 Media Attribute (a): rtpmap:101 telephone-event/8000 Media Attribute (a): fmtp:101 0-15 |
| 14. | Internet Protocol, Src Addr: 10.1.71.172 (10.1.71.172), Dst Addr: 10.1.70.249 (10.1.70.249) User Datagram Protocol, Src Port: 5060 (5060), Dst Port: 5060 (5060) Session Initiation Protocol Status line: SIP/2.0 200 OK Message Header From: "pat" <sip:pat@10.1.70.249:5060>;tag=zip4x4_3110815550-42 To: "ray" <sip:ray@zultys.com:5060>;tag=zip4x4_4000276508-43 Call-ID: 4000276057-43 CSeq: 2 INVITE Via: SIP/2.0/UDP 10.1.70.249;branch=z9hG4bK46ef.18da0543.0,SIP/2.0/UDP 10.1.71.187:5060 Record-Route: <sip:pat@10.1.70.249;ftag=zip4x4_3110815550-42;lr=on> Contact: ray <sip:ray@10.1.71.172:5060> Allow: INVITE,BYE,CANCEL,ACK,OPTIONS,REGISTER,NOTIFY,MESSAGE,REFER Content-Type: application/sdp Content-Length: 205 Session Description Protocol Session Description Protocol Version (v): 0 Owner/Creator, Session Id (o): ZIP4x4-ray-1.2.11 0 0 IN IP4 10.1.71.172 Session Name (s): zultys media Connection Information (c): IN IP4 0.0.0.0 Time Description, active time (t): 0 0 Media Description, name and address (m): audio 19996 RTP/AVP 0 101 Media Attribute (a): rtpmap:0 PCMU/8000 Media Attribute (a): ptime:20 Media Attribute (a): rtpmap:101 telephone-event/8000 Media Attribute (a): fmtp:101 0-15 |
| 15. | Internet Protocol, Src Addr: 10.1.70.249 (10.1.70.249), Dst Addr: 10.1.71.187 (10.1.71.187) User Datagram Protocol, Src Port: 5060 (5060), Dst Port: 5060 (5060) Session Initiation Protocol Status line: SIP/2.0 200 OK Message Header From: "pat" <sip:pat@10.1.70.249:5060>;tag=zip4x4_3110815550-42 To: "ray" <sip:ray@zultys.com:5060>;tag=zip4x4_4000276508-43 Call-ID: 4000276057-43 CSeq: 2 INVITE Via: SIP/2.0/UDP 10.1.71.187:5060 Record-Route: <sip:pat@10.1.70.249;ftag=zip4x4_3110815550-42;lr=on> Contact: ray <sip:ray@10.1.71.172:5060> Allow: INVITE,BYE,CANCEL,ACK,OPTIONS,REGISTER,NOTIFY,MESSAGE,REFER Content-Type: application/sdp Content-Length: 205 Session Description Protocol Session Description Protocol Version (v): 0 Owner/Creator, Session Id (o): ZIP4x4-ray-1.2.11 0 0 IN IP4 10.1.71.172 Session Name (s): zultys media Connection Information (c): IN IP4 0.0.0.0 Time Description, active time (t): 0 0 Media Description, name and address (m): audio 19996 RTP/AVP 0 101 Media Attribute (a): rtpmap:0 PCMU/8000 Media Attribute (a): ptime:20 Media Attribute (a): rtpmap:101 telephone-event/8000 Media Attribute (a): fmtp:101 0-15 |
| 16. | Internet Protocol, Src Addr: 10.1.71.187 (10.1.71.187), Dst Addr: 10.1.70.249 (10.1.70.249) User Datagram Protocol, Src Port: 5060 (5060), Dst Port: 5060 (5060) Session Initiation Protocol Request line: ACK sip:pat@10.1.70.249;ftag=zip4x4_3110815550-42;lr=on SIP/2.0 Message Header From: "pat" <sip:pat@10.1.70.249:5060>;tag=zip4x4_3110815550-42 To: "ray" <sip:ray@zultys.com:5060>;tag=zip4x4_4000276508-43 Call-ID: 4000276057-43 CSeq: 2 ACK Via: SIP/2.0/UDP 10.1.71.187:5060 Contact: pat <sip:pat@10.1.71.187:5060> Route: <sip:ray@10.1.71.172:5060> Content-Length: 0 |
| 17. | Internet Protocol, Src Addr: 10.1.70.249 (10.1.70.249), Dst Addr: 10.1.71.172 (10.1.71.172) User Datagram Protocol, Src Port: 5060 (5060), Dst Port: 5060 (5060) Session Initiation Protocol Request line: ACK sip:ray@10.1.71.172:5060 SIP/2.0 Message Header Max-Forwards: 10 Record-Route: <sip:pat@10.1.70.249;ftag=zip4x4_3110815550-42;lr=on> From: "pat" <sip:pat@10.1.70.249:5060>;tag=zip4x4_3110815550-42 To: "ray" <sip:ray@zultys.com:5060>;tag=zip4x4_4000276508-43 Call-ID: 4000276057-43 CSeq: 2 ACK Via: SIP/2.0/UDP 10.1.70.249;branch=0 Via: SIP/2.0/UDP 10.1.71.187:5060 Contact: pat <sip:pat@10.1.71.187:5060> Content-Length: 0 |
| 18. | Internet Protocol, Src Addr: 10.1.71.187 (10.1.71.187), Dst Addr: 10.1.70.249 (10.1.70.249) User Datagram Protocol, Src Port: 5060 (5060), Dst Port: 5060 (5060) Session Initiation Protocol Request line: REFER sip:pat@10.1.70.249;ftag=zip4x4_3110815550-42;lr=on SIP/2.0 Message Header From: "pat" <sip:pat@10.1.70.249:5060>;tag=zip4x4_3110815550-42 To: "ray" <sip:ray@zultys.com:5060>;tag=zip4x4_4000276508-43 Call-ID: 4000276057-43 CSeq: 3 REFER Via: SIP/2.0/UDP 10.1.71.187:5060 Contact: pat <sip:pat@10.1.71.187:5060> Allow: INVITE,BYE,CANCEL,ACK,OPTIONS,REGISTER,NOTIFY,MESSAGE,REFER User-Agent: Zultys ZIP4x4 1.2.14 Route: <sip:ray@10.1.71.172:5060> Refer-To: sip:mx25_1@10.1.70.249 Referred-By: "ray" <sip:ray@zultys.com:5060> Content-Length: 0 |
| 19. | Internet Protocol, Src Addr: 10.1.70.249 (10.1.70.249), Dst Addr: 10.1.71.172 (10.1.71.172) User Datagram Protocol, Src Port: 5060 (5060), Dst Port: 5060 (5060) Session Initiation Protocol Request line: REFER sip:ray@10.1.71.172:5060 SIP/2.0 Message Header Max-Forwards: 10 Record-Route: <sip:pat@10.1.70.249;ftag=zip4x4_3110815550-42;lr=on> From: "pat" <sip:pat@10.1.70.249:5060>;tag=zip4x4_3110815550-42 To: "ray" <sip:ray@zultys.com:5060>;tag=zip4x4_4000276508-43 Call-ID: 4000276057-43 CSeq: 3 REFER Via: SIP/2.0/UDP 10.1.70.249;branch=z9hG4bK56ef.1bd77155.0 Via: SIP/2.0/UDP 10.1.71.187:5060 Contact: pat <sip:pat@10.1.71.187:5060> Allow: INVITE,BYE,CANCEL,ACK,OPTIONS,REGISTER,NOTIFY,MESSAGE,REFER User-Agent: Zultys ZIP4x4 1.2.14 Refer-To: sip:mx25_1@10.1.70.249 Referred-By: "ray" <sip:ray@zultys.com:5060> Content-Length: 0 |
| 20. | Internet Protocol, Src Addr: 10.1.71.172 (10.1.71.172), Dst Addr: 10.1.70.249 (10.1.70.249) User Datagram Protocol, Src Port: 5060 (5060), Dst Port: 5060 (5060) Session Initiation Protocol Request line: INVITE sip:mx25_1@10.1.70.249:5060 SIP/2.0 Message Header From: "ray" <sip:ray@zultys.com:5060>;tag=zip4x4_4045269138-43 To: "mx25_1" <sip:mx25_1@10.1.70.249:5060> Call-ID: 4045268654-43 CSeq: 1 INVITE Via: SIP/2.0/UDP 10.1.71.172:5060 Contact: ray <sip:ray@10.1.71.172:5060> Allow: INVITE,BYE,CANCEL,ACK,OPTIONS,REGISTER,NOTIFY,MESSAGE,REFER User-Agent: Zultys ZIP4x4 1.2.11 Content-Type: application/sdp Content-Length: 259 Session Description Protocol Session Description Protocol Version (v): 0 Owner/Creator, Session Id (o): ZIP4x4-ray-1.2.11 0 0 IN IP4 10.1.71.172 Session Name (s): zultys media Connection Information (c): IN IP4 10.1.71.172 Time Description, active time (t): 0 0 Media Description, name and address (m): audio 19996 RTP/AVP 0 8 18 101 Media Attribute (a): rtpmap:0 PCMU/8000 Media Attribute (a): rtpmap:8 PCMA/8000 Media Attribute (a): rtpmap:18 G729/8000 Media Attribute (a): rtpmap:101 telephone-event/8000 Media Attribute (a): fmtp:101 0-15 Media Attribute (a): ptime:20 |
| 21. | Internet Protocol, Src Addr: 10.1.70.249 (10.1.70.249), Dst Addr: 10.1.71.172 (10.1.71.172) User Datagram Protocol, Src Port: 5060 (5060), Dst Port: 5060 (5060) Session Initiation Protocol Status line: SIP/2.0 100 trying -- your call is important to us Message Header From: "ray" <sip:ray@zultys.com:5060>;tag=zip4x4_4045269138-43 To: "mx25_1" <sip:mx25_1@10.1.70.249:5060> Call-ID: 4045268654-43 CSeq: 1 INVITE Via: SIP/2.0/UDP 10.1.71.172:5060 Server: Sip EXpress router (0.8.12 (i386/linux)) Content-Length: 0 Warning: 392 10.1.70.249:5060 "Noisy feedback tells: pid=7848 req_src_ip=10.1.71.172 req_src_port=5060 in_uri=sip:mx25_1@10.1.70.249:5060 out_uri=sip:MX25_1@10.1.71.254:5060 via_cnt==1" |
| 22. | Internet Protocol, Src Addr: 10.1.70.249 (10.1.70.249), Dst Addr: 10.1.71.254 (10.1.71.254) User Datagram Protocol, Src Port: 5060 (5060), Dst Port: 5060 (5060) Session Initiation Protocol Request line: INVITE sip:MX25_1@10.1.71.254:5060 SIP/2.0 Message Header Max-Forwards: 10 Record-Route: <sip:mx25_1@10.1.70.249;ftag=zip4x4_4045269138-43;lr=on> From: "ray" <sip:ray@zultys.com:5060>;tag=zip4x4_4045269138-43 To: "mx25_1" <sip:mx25_1@10.1.70.249:5060> Call-ID: 4045268654-43 CSeq: 1 INVITE Via: SIP/2.0/UDP 10.1.70.249;branch=z9hG4bKf19.b8aea453.0 Via: SIP/2.0/UDP 10.1.71.172:5060 Contact: ray <sip:ray@10.1.71.172:5060> Allow: INVITE,BYE,CANCEL,ACK,OPTIONS,REGISTER,NOTIFY,MESSAGE,REFER User-Agent: Zultys ZIP4x4 1.2.11 Content-Type: application/sdp Content-Length: 259 P-hint: USRLOC Session Description Protocol Session Description Protocol Version (v): 0 Owner/Creator, Session Id (o): ZIP4x4-ray-1.2.11 0 0 IN IP4 10.1.71.172 Session Name (s): zultys media Connection Information (c): IN IP4 10.1.71.172 Time Description, active time (t): 0 0 Media Description, name and address (m): audio 19996 RTP/AVP 0 8 18 101 Media Attribute (a): rtpmap:0 PCMU/8000 Media Attribute (a): rtpmap:8 PCMA/8000 Media Attribute (a): rtpmap:18 G729/8000 Media Attribute (a): rtpmap:101 telephone-event/8000 Media Attribute (a): fmtp:101 0-15 Media Attribute (a): ptime:20 |
| 23. | Internet Protocol, Src Addr: 10.1.71.172 (10.1.71.172), Dst Addr: 10.1.70.249 (10.1.70.249) User Datagram Protocol, Src Port: 5060 (5060), Dst Port: 5060 (5060) Session Initiation Protocol Status line: SIP/2.0 202 Accepted Message Header From: "pat" <sip:pat@10.1.70.249:5060>;tag=zip4x4_3110815550-42 To: "ray" <sip:ray@zultys.com:5060>;tag=zip4x4_4000276508-43 Call-ID: 4000276057-43 CSeq: 3 REFER Via: SIP/2.0/UDP 10.1.70.249;branch=z9hG4bK56ef.1bd77155.0,SIP/2.0/UDP 10.1.71.187:5060 Record-Route: <sip:pat@10.1.70.249;ftag=zip4x4_3110815550-42;lr=on> Contact: ray <sip:ray@10.1.71.172:5060> Content-Length: 0 |
| 24. | Internet Protocol, Src Addr: 10.1.70.249 (10.1.70.249), Dst Addr: 10.1.71.187 (10.1.71.187) User Datagram Protocol, Src Port: 5060 (5060), Dst Port: 5060 (5060) Session Initiation Protocol Status line: SIP/2.0 202 Accepted Message Header From: "pat" <sip:pat@10.1.70.249:5060>;tag=zip4x4_3110815550-42 To: "ray" <sip:ray@zultys.com:5060>;tag=zip4x4_4000276508-43 Call-ID: 4000276057-43 CSeq: 3 REFER Via: SIP/2.0/UDP 10.1.71.187:5060 Record-Route: <sip:pat@10.1.70.249;ftag=zip4x4_3110815550-42;lr=on> Contact: ray <sip:ray@10.1.71.172:5060> Content-Length: 0 |
| 25. | Internet Protocol, Src Addr: 10.1.71.187 (10.1.71.187), Dst Addr: 10.1.70.249 (10.1.70.249) User Datagram Protocol, Src Port: 5060 (5060), Dst Port: 5060 (5060) Session Initiation Protocol Request line: BYE sip:pat@10.1.70.249;ftag=zip4x4_3110815550-42;lr=on SIP/2.0 Message Header From: "pat" <sip:pat@10.1.70.249:5060>;tag=zip4x4_3110815550-42 To: "ray" <sip:ray@zultys.com:5060>;tag=zip4x4_4000276508-43 Call-ID: 4000276057-43 CSeq: 4 BYE Via: SIP/2.0/UDP 10.1.71.187:5060 Route: <sip:ray@10.1.71.172:5060> Content-Length: 0 |
| 26. | Internet Protocol, Src Addr: 10.1.70.249 (10.1.70.249), Dst Addr: 10.1.71.172 (10.1.71.172) User Datagram Protocol, Src Port: 5060 (5060), Dst Port: 5060 (5060) Session Initiation Protocol Request line: BYE sip:ray@10.1.71.172:5060 SIP/2.0 Message Header Max-Forwards: 10 Record-Route: <sip:pat@10.1.70.249;ftag=zip4x4_3110815550-42;lr=on> From: "pat" <sip:pat@10.1.70.249:5060>;tag=zip4x4_3110815550-42 To: "ray" <sip:ray@zultys.com:5060>;tag=zip4x4_4000276508-43 Call-ID: 4000276057-43 CSeq: 4 BYE Via: SIP/2.0/UDP 10.1.70.249;branch=z9hG4bK26ef.d2d629a.0 Via: SIP/2.0/UDP 10.1.71.187:5060 Content-Length: 0 |
| 27. | Internet Protocol, Src Addr: 10.1.71.254 (10.1.71.254), Dst Addr: 10.1.70.249 (10.1.70.249) User Datagram Protocol, Src Port: 5060 (5060), Dst Port: 5060 (5060) Session Initiation Protocol Status line: SIP/2.0 100 Trying Message Header From: "ray" <sip:ray@zultys.com:5060>;tag=zip4x4_4045269138-43 To: "mx25_1" <sip:mx25_1@10.1.70.249:5060>;tag=25-nvs--1035999994124_1462514997-124 Call-ID: 4045268654-43 CSeq: 1 INVITE Via: SIP/2.0/UDP 10.1.70.249;branch=z9hG4bKf19.b8aea453.0,SIP/2.0/UDP 10.1.71.172:5060 Record-Route: <sip:mx25_1@10.1.70.249;ftag=zip4x4_4045269138-43;lr=on> Server: Zultys MX25 Content-Length: 0 |
| 28. | Internet Protocol, Src Addr: 10.1.71.172 (10.1.71.172), Dst Addr: 10.1.70.249 (10.1.70.249) User Datagram Protocol, Src Port: 5060 (5060), Dst Port: 5060 (5060) Session Initiation Protocol Status line: SIP/2.0 200 OK Message Header From: "pat" <sip:pat@10.1.70.249:5060>;tag=zip4x4_3110815550-42 To: "ray" <sip:ray@zultys.com:5060>;tag=zip4x4_4000276508-43 Call-ID: 4000276057-43 CSeq: 4 BYE Via: SIP/2.0/UDP 10.1.70.249;branch=z9hG4bK26ef.d2d629a.0,SIP/2.0/UDP 10.1.71.187:5060 Record-Route: <sip:pat@10.1.70.249;ftag=zip4x4_3110815550-42;lr=on> Contact: ray <sip:ray@10.1.71.172:5060> Allow: INVITE,BYE,CANCEL,ACK,OPTIONS,REGISTER,NOTIFY,MESSAGE,REFER Content-Length: 0 |
| 29. | Internet Protocol, Src Addr: 10.1.70.249 (10.1.70.249), Dst Addr: 10.1.71.187 (10.1.71.187) User Datagram Protocol, Src Port: 5060 (5060), Dst Port: 5060 (5060) Session Initiation Protocol Status line: SIP/2.0 200 OK Message Header From: "pat" <sip:pat@10.1.70.249:5060>;tag=zip4x4_3110815550-42 To: "ray" <sip:ray@zultys.com:5060>;tag=zip4x4_4000276508-43 Call-ID: 4000276057-43 CSeq: 4 BYE Via: SIP/2.0/UDP 10.1.71.187:5060 Record-Route: <sip:pat@10.1.70.249;ftag=zip4x4_3110815550-42;lr=on> Contact: ray <sip:ray@10.1.71.172:5060> Allow: INVITE,BYE,CANCEL,ACK,OPTIONS,REGISTER,NOTIFY,MESSAGE,REFER Content-Length: 0 |
| 30. | Internet Protocol, Src Addr: 10.1.71.254 (10.1.71.254), Dst Addr: 10.1.70.249 (10.1.70.249) User Datagram Protocol, Src Port: 5060 (5060), Dst Port: 5060 (5060) Session Initiation Protocol Status line: SIP/2.0 180 Ringing Message Header From: "ray" <sip:ray@zultys.com:5060>;tag=zip4x4_4045269138-43 To: "mx25_1" <sip:mx25_1@10.1.70.249:5060>;tag=25-nvs--1035999994124_1462514997-124 Call-ID: 4045268654-43 CSeq: 1 INVITE Via: SIP/2.0/UDP 10.1.70.249;branch=z9hG4bKf19.b8aea453.0,SIP/2.0/UDP 10.1.71.172:5060 Record-Route: <sip:mx25_1@10.1.70.249;ftag=zip4x4_4045269138-43;lr=on> Server: Zultys MX25 Content-Length: 0 |
| 31. | Internet Protocol, Src Addr: 10.1.70.249 (10.1.70.249), Dst Addr: 10.1.71.172 (10.1.71.172) User Datagram Protocol, Src Port: 5060 (5060), Dst Port: 5060 (5060) Session Initiation Protocol Status line: SIP/2.0 180 Ringing Message Header From: "ray" <sip:ray@zultys.com:5060>;tag=zip4x4_4045269138-43 To: "mx25_1" <sip:mx25_1@10.1.70.249:5060>;tag=25-nvs--1035999994124_1462514997-124 Call-ID: 4045268654-43 CSeq: 1 INVITE Via: SIP/2.0/UDP 10.1.71.172:5060 Record-Route: <sip:mx25_1@10.1.70.249;ftag=zip4x4_4045269138-43;lr=on> Server: Zultys MX25 Content-Length: 0 |
| 32. | Internet Protocol, Src Addr: 10.1.71.254 (10.1.71.254), Dst Addr: 10.1.70.249 (10.1.70.249) User Datagram Protocol, Src Port: 5060 (5060), Dst Port: 5060 (5060) Session Initiation Protocol Status line: SIP/2.0 200 OK Message Header From: "ray" <sip:ray@zultys.com:5060>;tag=zip4x4_4045269138-43 To: "mx25_1" <sip:mx25_1@10.1.70.249:5060>;tag=25-nvs--1035999994124_1462514997-124 Call-ID: 4045268654-43 CSeq: 1 INVITE Session-Expires: 3600;refresher=uas Via: SIP/2.0/UDP 10.1.70.249;branch=z9hG4bKf19.b8aea453.0,SIP/2.0/UDP 10.1.71.172:5060 Record-Route: <sip:mx25_1@10.1.70.249;ftag=zip4x4_4045269138-43;lr=on> Contact: sip:mx25_1@10.1.71.254:5060 Allow: INVITE,ACK,CANCEL,BYE,REGISTER,OPTIONS,NOTIFY,SUBSCRIBE,REFER,MESSAGE Content-Type: application/sdp Server: Zultys MX25 Content-Length: 244 Session Description Protocol Session Description Protocol Version (v): 0 Owner/Creator, Session Id (o): MX25 1026854232 0 IN IP4 10.1.71.254 Session Name (s): - Connection Information (c): IN IP4 10.1.71.254 Time Description, active time (t): 0 0 Media Description, name and address (m): audio 30010 RTP/AVP 0 8 18 101 Media Attribute (a): rtpmap:0 PCMU/8000 Media Attribute (a): rtpmap:8 PCMA/8000 Media Attribute (a): rtpmap:18 G729/8000 Media Attribute (a): rtpmap:101 telephone-event/8000 Media Attribute (a): fmtp:101 0-15 Media Attribute (a): ptime:20 |
| 33. | Internet Protocol, Src Addr: 10.1.70.249 (10.1.70.249), Dst Addr: 10.1.71.172 (10.1.71.172) User Datagram Protocol, Src Port: 5060 (5060), Dst Port: 5060 (5060) Session Initiation Protocol Status line: SIP/2.0 200 OK Message Header From: "ray" <sip:ray@zultys.com:5060>;tag=zip4x4_4045269138-43 To: "mx25_1" <sip:mx25_1@10.1.70.249:5060>;tag=25-nvs--1035999994124_1462514997-124 Call-ID: 4045268654-43 CSeq: 1 INVITE Session-Expires: 3600;refresher=uas Via: SIP/2.0/UDP 10.1.71.172:5060 Record-Route: <sip:mx25_1@10.1.70.249;ftag=zip4x4_4045269138-43;lr=on> Contact: sip:mx25_1@10.1.71.254:5060 Allow: INVITE,ACK,CANCEL,BYE,REGISTER,OPTIONS,NOTIFY,SUBSCRIBE,REFER,MESSAGE Content-Type: application/sdp Server: Zultys MX25 Content-Length: 244 Session Description Protocol Session Description Protocol Version (v): 0 Owner/Creator, Session Id (o): MX25 1026854232 0 IN IP4 10.1.71.254 Session Name (s): - Connection Information (c): IN IP4 10.1.71.254 Time Description, active time (t): 0 0 Media Description, name and address (m): audio 30010 RTP/AVP 0 8 18 101 Media Attribute (a): rtpmap:0 PCMU/8000 Media Attribute (a): rtpmap:8 PCMA/8000 Media Attribute (a): rtpmap:18 G729/8000 Media Attribute (a): rtpmap:101 telephone-event/8000 Media Attribute (a): fmtp:101 0-15 Media Attribute (a): ptime:20 |
| 34. | Internet Protocol, Src Addr: 10.1.71.172 (10.1.71.172), Dst Addr: 10.1.70.249 (10.1.70.249) User Datagram Protocol, Src Port: 5060 (5060), Dst Port: 5060 (5060) Session Initiation Protocol Request line: ACK sip:mx25_1@10.1.70.249:5060 SIP/2.0 Message Header From: "ray" <sip:ray@zultys.com:5060>;tag=zip4x4_4045269138-43 To: "mx25_1" <sip:mx25_1@10.1.70.249:5060>;tag=25-nvs--1035999994124_1462514997-124 Call-ID: 4045268654-43 CSeq: 1 ACK Via: SIP/2.0/UDP 10.1.71.172:5060 Contact: ray <sip:ray@10.1.71.172:5060> Route: <sip:mx25_1@10.1.70.249;ftag=zip4x4_4045269138-43;lr=on>,<sip:mx25_1@10.1.71.254:5060> Content-Length: 0 |
| 35. | Internet Protocol, Src Addr: 10.1.70.249 (10.1.70.249), Dst Addr: 10.1.71.254 (10.1.71.254) User Datagram Protocol, Src Port: 5060 (5060), Dst Port: 5060 (5060) Session Initiation Protocol Request line: ACK sip:mx25_1@10.1.71.254:5060 SIP/2.0 Message Header Max-Forwards: 10 Record-Route: <sip:mx25_1@10.1.70.249;ftag=zip4x4_4045269138-43;lr=on> From: "ray" <sip:ray@zultys.com:5060>;tag=zip4x4_4045269138-43 To: "mx25_1" <sip:mx25_1@10.1.70.249:5060>;tag=25-nvs--1035999994124_1462514997-124 Call-ID: 4045268654-43 CSeq: 1 ACK Via: SIP/2.0/UDP 10.1.70.249;branch=0 Via: SIP/2.0/UDP 10.1.71.172:5060 Contact: ray <sip:ray@10.1.71.172:5060> Content-Length: 0 |

# Call with Authentication

Figure 7 shows a SIP call made using digest authentication. Note that the device registration was also authenticated by the proxy (SER in this case).

Figure 7: Authenticated Registration and Call

| 1. | Internet Protocol, Src Addr: 10.1.71.131 (10.1.71.131), Dst Addr: 10.1.70.249 (10.1.70.249) User Datagram Protocol, Src Port: 5060 (5060), Dst Port: 5060 (5060) Session Initiation Protocol Request line: REGISTER sip:10.1.70.249:5060 SIP/2.0 Message Header From: "mike" <sip:mike@10.1.70.249:5060>;tag=zip4x4_800297596-22 To: "mike" <sip:mike@10.1.70.249:5060> Call-ID: 800296992-22 CSeq: 1 REGISTER Via: SIP/2.0/UDP 10.1.71.131:5060 Contact: mike <sip:mike@10.1.71.131:5060> Allow: INVITE,BYE,CANCEL,ACK,OPTIONS,REGISTER,NOTIFY,MESSAGE,REFER User-Agent: Zultys ZIP4x4 1.1.65 Expires: 3600 Content-Length: 0 |
| --- | --- |
| 2. | Internet Protocol, Src Addr: 10.1.70.249 (10.1.70.249), Dst Addr: 10.1.71.131 (10.1.71.131) User Datagram Protocol, Src Port: 5060 (5060), Dst Port: 5060 (5060) Session Initiation Protocol Status line: SIP/2.0 401 Unauthorized Message Header From: "mike" <sip:mike@10.1.70.249:5060>;tag=zip4x4_800297596-22 To: "mike" <sip:mike@10.1.70.249:5060>;tag=e3c7b26f770301a96cbe1ba383aff22a.c5e2 Call-ID: 800296992-22 CSeq: 1 REGISTER Via: SIP/2.0/UDP 10.1.71.131:5060 WWW-Authenticate: Digest realm="sqalab.com", nonce="40da3b6ca53371725ec2ae158306739f19214f25" Server: Sip EXpress router (0.8.12 (i386/linux)) Content-Length: 0 Warning: 392 10.1.70.249:5060 "Noisy feedback tells: pid=23634 req_src_ip=10.1.71.131 req_src_port=5060 in_uri=sip:10.1.70.249:5060 out_uri=sip:10.1.70.249:5060 via_cnt==1" |
| 3. | Internet Protocol, Src Addr: 10.1.71.131 (10.1.71.131), Dst Addr: 10.1.70.249 (10.1.70.249) User Datagram Protocol, Src Port: 5060 (5060), Dst Port: 5060 (5060) Session Initiation Protocol Request line: REGISTER sip:10.1.70.249:5060 SIP/2.0 Message Header From: "mike" <sip:mike@10.1.70.249:5060>;tag=zip4x4_800297596-22 To: "mike" <sip:mike@10.1.70.249:5060> Call-ID: 800296992-22 CSeq: 2 REGISTER Via: SIP/2.0/UDP 10.1.71.131:5060 Contact: mike <sip:mike@10.1.71.131:5060> Allow: INVITE,BYE,CANCEL,ACK,OPTIONS,REGISTER,NOTIFY,MESSAGE,REFER User-Agent: Zultys ZIP4x4 1.1.65 Expires: 3600 Authorization: Digest username="mike",realm="sqalab.com", nonce="40da3b6ca53371725ec2 ae158306739f19214f25", uri="sip:10.1.70.249:5060",response="073269f7bd766b0aa72472a4eb11b329",algorithm=md5 Content-Length: 0 |
| 4. | Internet Protocol, Src Addr: 10.1.70.249 (10.1.70.249), Dst Addr: 10.1.71.131 (10.1.71.131) User Datagram Protocol, Src Port: 5060 (5060), Dst Port: 5060 (5060) Session Initiation Protocol Status line: SIP/2.0 200 OK Message Header From: "mike" <sip:mike@10.1.70.249:5060>;tag=zip4x4_800297596-22 To: "mike" <sip:mike@10.1.70.249:5060>;tag=e3c7b26f770301a96cbe1ba383aff22a.c5e2 Call-ID: 800296992-22 CSeq: 2 REGISTER Via: SIP/2.0/UDP 10.1.71.131:5060 Contact: <sip:mike@10.1.71.131:5060>;q=0.00;expires=3600 Server: Sip EXpress router (0.8.12 (i386/linux)) Content-Length: 0 Warning: 392 10.1.70.249:5060 "Noisy feedback tells: pid=23634 req_src_ip=10.1.71.131 req_src_port=5060 in_uri=sip:10.1.70.249:5060 out_uri=sip:10.1.70.249:5060 via_cnt==1" |
| 5. | Internet Protocol, Src Addr: 10.1.71.133 (10.1.71.133), Dst Addr: 10.1.70.249 (10.1.70.249) User Datagram Protocol, Src Port: 5060 (5060), Dst Port: 5060 (5060) Session Initiation Protocol Request line: REGISTER sip:10.1.70.249:5060 SIP/2.0 Message Header From: "pat" <sip:pat@10.1.70.249:5060>;tag=zip4x4_3012362611-22 To: "pat" <sip:pat@10.1.70.249:5060> Call-ID: 3012362048-22 CSeq: 1 REGISTER Via: SIP/2.0/UDP 10.1.71.133:5060 Contact: pat <sip:pat@10.1.71.133:5060> Allow: INVITE,BYE,CANCEL,ACK,OPTIONS,REGISTER,NOTIFY,MESSAGE,REFER User-Agent: Zultys ZIP4x4 1.1.65 Expires: 3600 Content-Length: 0 |
| 6. | Internet Protocol, Src Addr: 10.1.70.249 (10.1.70.249), Dst Addr: 10.1.71.133 (10.1.71.133) User Datagram Protocol, Src Port: 5060 (5060), Dst Port: 5060 (5060) Session Initiation Protocol Status line: SIP/2.0 401 Unauthorized Message Header From: "pat" <sip:pat@10.1.70.249:5060>;tag=zip4x4_3012362611-22 To: "pat" <sip:pat@10.1.70.249:5060>;tag=e3c7b26f770301a96cbe1ba383aff22a.4d83 Call-ID: 3012362048-22 CSeq: 1 REGISTER Via: SIP/2.0/UDP 10.1.71.133:5060 WWW-Authenticate: Digest realm="sqalab.com", nonce="40da3b74b16f8c3dced50447f7510bb17c5b0de4" Server: Sip EXpress router (0.8.12 (i386/linux)) Content-Length: 0 Warning: 392 10.1.70.249:5060 "Noisy feedback tells: pid=23634 req_src_ip=10.1.71.133 req_src_port=5060 in_uri=sip:10.1.70.249:5060 out_uri=sip:10.1.70.249:5060 via_cnt==1" |
| 7. | Internet Protocol, Src Addr: 10.1.71.133 (10.1.71.133), Dst Addr: 10.1.70.249 (10.1.70.249) User Datagram Protocol, Src Port: 5060 (5060), Dst Port: 5060 (5060) Session Initiation Protocol Request line: REGISTER sip:10.1.70.249:5060 SIP/2.0 Message Header From: "pat" <sip:pat@10.1.70.249:5060>;tag=zip4x4_3012362611-22 To: "pat" <sip:pat@10.1.70.249:5060> Call-ID: 3012362048-22 CSeq: 2 REGISTER Via: SIP/2.0/UDP 10.1.71.133:5060 Contact: pat <sip:pat@10.1.71.133:5060> Allow: INVITE,BYE,CANCEL,ACK,OPTIONS,REGISTER,NOTIFY,MESSAGE,REFER User-Agent: Zultys ZIP4x4 1.1.65 Expires: 3600 Authorization: Digest username="pat",realm="sqalab.com",nonce="40da3b74b16f8c3dced50447f7510bb17c5b0de4", uri="sip:10.1.70.249:5060",response="e6df20063469b200b048d2f4eb46c9af",algorithm=md5 Content-Length: 0 |
| 8. | Internet Protocol, Src Addr: 10.1.70.249 (10.1.70.249), Dst Addr: 10.1.71.133 (10.1.71.133) User Datagram Protocol, Src Port: 5060 (5060), Dst Port: 5060 (5060) Session Initiation Protocol Status line: SIP/2.0 200 OK Message Header From: "pat" <sip:pat@10.1.70.249:5060>;tag=zip4x4_3012362611-22 To: "pat" <sip:pat@10.1.70.249:5060>;tag=e3c7b26f770301a96cbe1ba383aff22a.4d83 Call-ID: 3012362048-22 CSeq: 2 REGISTER Via: SIP/2.0/UDP 10.1.71.133:5060 Contact: <sip:pat@10.1.71.133:5060>;q=0.00;expires=3600 Server: Sip EXpress router (0.8.12 (i386/linux)) Content-Length: 0 Warning: 392 10.1.70.249:5060 "Noisy feedback tells: pid=23634 req_src_ip=10.1.71.133 req_src_port=5060 in_uri=sip:10.1.70.249:5060 out_uri=sip:10.1.70.249:5060 via_cnt==1" |
| 9. | Internet Protocol, Src Addr: 10.1.71.131 (10.1.71.131), Dst Addr: 10.1.70.249 (10.1.70.249) User Datagram Protocol, Src Port: 5060 (5060), Dst Port: 5060 (5060) Session Initiation Protocol Request line: INVITE sip:pat@10.1.70.249:5060 SIP/2.0 Message Header From: "mike" <sip:mike@10.1.70.249:5060>;tag=zip4x4_810445264-42 To: "pat" <sip:pat@10.1.70.249:5060> Call-ID: 810444843-42 CSeq: 1 INVITE Via: SIP/2.0/UDP 10.1.71.131:5060 Contact: mike <sip:mike@10.1.71.131:5060> Allow: INVITE,BYE,CANCEL,ACK,OPTIONS,REGISTER,NOTIFY,MESSAGE,REFER User-Agent: Zultys ZIP4x4 1.1.65 Content-Type: application/sdp Content-Length: 260 Session Description Protocol Session Description Protocol Version (v): 0 Owner/Creator, Session Id (o): ZIP4x4-mike-1.1.65 0 0 IN IP4 10.1.71.131 Session Name (s): zultys media Connection Information (c): IN IP4 10.1.71.131 Time Description, active time (t): 0 0 Media Description, name and address (m): audio 20000 RTP/AVP 0 8 18 101 Media Attribute (a): rtpmap:0 PCMU/8000 Media Attribute (a): rtpmap:8 PCMA/8000 Media Attribute (a): rtpmap:18 G729/8000 Media Attribute (a): rtpmap:101 telephone-event/8000 Media Attribute (a): fmtp:101 0-15 Media Attribute (a): ptime:20 |
| 10. | Internet Protocol, Src Addr: 10.1.70.249 (10.1.70.249), Dst Addr: 10.1.71.131 (10.1.71.131) User Datagram Protocol, Src Port: 5060 (5060), Dst Port: 5060 (5060) Session Initiation Protocol Status line: SIP/2.0 407 Proxy Authentication Required Message Header From: "mike" <sip:mike@10.1.70.249:5060>;tag=zip4x4_810445264-42 To: "pat" <sip:pat@10.1.70.249:5060>;tag=e3c7b26f770301a96cbe1ba383aff22a.c5e2 Call-ID: 810444843-42 CSeq: 1 INVITE Via: SIP/2.0/UDP 10.1.71.131:5060 Proxy-Authenticate: Digest realm="sqalab.com", nonce="40da3b76753c8954f01424844614c82531a99e22" Server: Sip EXpress router (0.8.12 (i386/linux)) Content-Length: 0 Warning: 392 10.1.70.249:5060 "Noisy feedback tells: pid=23634 req_src_ip=10.1.71.131 req_src_port=5060 in_uri=sip:pat@10.1.70.249:5060 out_uri=sip:pat@10.1.70.249:5060 via_cnt==1" |
| 11. | Internet Protocol, Src Addr: 10.1.71.131 (10.1.71.131), Dst Addr: 10.1.70.249 (10.1.70.249) User Datagram Protocol, Src Port: 5060 (5060), Dst Port: 5060 (5060) Session Initiation Protocol Request line: ACK sip:pat@10.1.70.249:5060 SIP/2.0 Message Header From: "mike" <sip:mike@10.1.70.249:5060>;tag=zip4x4_810445264-42 To: "pat" <sip:pat@10.1.70.249:5060>;tag=e3c7b26f770301a96cbe1ba383aff22a.c5e2 Call-ID: 810444843-42 CSeq: 1 ACK Via: SIP/2.0/UDP 10.1.71.131:5060 Contact: mike <sip:mike@10.1.71.131:5060> Content-Length: 0 |
| 12. | Internet Protocol, Src Addr: 10.1.71.131 (10.1.71.131), Dst Addr: 10.1.70.249 (10.1.70.249) User Datagram Protocol, Src Port: 5060 (5060), Dst Port: 5060 (5060) Session Initiation Protocol Request line: INVITE sip:pat@10.1.70.249:5060 SIP/2.0 Message Header From: "mike" <sip:mike@10.1.70.249:5060>;tag=zip4x4_810445264-42 To: "pat" <sip:pat@10.1.70.249:5060> Call-ID: 810444843-42 CSeq: 2 INVITE Via: SIP/2.0/UDP 10.1.71.131:5060 Contact: mike <sip:mike@10.1.71.131:5060> Allow: INVITE,BYE,CANCEL,ACK,OPTIONS,REGISTER,NOTIFY,MESSAGE,REFER User-Agent: Zultys ZIP4x4 1.1.65 Content-Type: application/sdp Proxy-Authorization: Digest username="mike",realm="sqalab.com", nonce="40da3b76753c8954f01424844614c82531a99e22", uri="sip:pat@10.1.70.249:5060",response="8bea621485a6105b571868763820f95f",algorithm=md5 Content-Length: 260 Session Description Protocol Session Description Protocol Version (v): 0 Owner/Creator, Session Id (o): ZIP4x4-mike-1.1.65 0 0 IN IP4 10.1.71.131 Session Name (s): zultys media Connection Information (c): IN IP4 10.1.71.131 Time Description, active time (t): 0 0 Media Description, name and address (m): audio 20000 RTP/AVP 0 8 18 101 Media Attribute (a): rtpmap:0 PCMU/8000 Media Attribute (a): rtpmap:8 PCMA/8000 Media Attribute (a): rtpmap:18 G729/8000 Media Attribute (a): rtpmap:101 telephone-event/8000 Media Attribute (a): fmtp:101 0-15 Media Attribute (a): ptime:20 |
| 13. | Internet Protocol, Src Addr: 10.1.70.249 (10.1.70.249), Dst Addr: 10.1.71.131 (10.1.71.131) User Datagram Protocol, Src Port: 5060 (5060), Dst Port: 5060 (5060) Session Initiation Protocol Status line: SIP/2.0 100 trying -- your call is important to us Message Header From: "mike" <sip:mike@10.1.70.249:5060>;tag=zip4x4_810445264-42 To: "pat" <sip:pat@10.1.70.249:5060> Call-ID: 810444843-42 CSeq: 2 INVITE Via: SIP/2.0/UDP 10.1.71.131:5060 Server: Sip EXpress router (0.8.12 (i386/linux)) Content-Length: 0 Warning: 392 10.1.70.249:5060 "Noisy feedback tells: pid=23634 req_src_ip=10.1.71.131 req_src_port=5060 in_uri=sip:pat@10.1.70.249:5060 out_uri=sip:pat@10.1.71.133:5060 via_cnt==1" |
| 14. | Internet Protocol, Src Addr: 10.1.70.249 (10.1.70.249), Dst Addr: 10.1.71.133 (10.1.71.133) User Datagram Protocol, Src Port: 5060 (5060), Dst Port: 5060 (5060) Session Initiation Protocol Request line: INVITE sip:pat@10.1.71.133:5060 SIP/2.0 Message Header Max-Forwards: 10 Record-Route: <sip:pat@10.1.70.249;ftag=zip4x4_810445264-42;lr=on> From: "mike" <sip:mike@10.1.70.249:5060>;tag=zip4x4_810445264-42 To: "pat" <sip:pat@10.1.70.249:5060> Call-ID: 810444843-42 CSeq: 2 INVITE Via: SIP/2.0/UDP 10.1.70.249;branch=z9hG4bKa798.0647c7d.0 Via: SIP/2.0/UDP 10.1.71.131:5060 Contact: mike <sip:mike@10.1.71.131:5060> Allow: INVITE,BYE,CANCEL,ACK,OPTIONS,REGISTER,NOTIFY,MESSAGE,REFER User-Agent: Zultys ZIP4x4 1.1.65 Content-Type: application/sdp Proxy-Authorization: Digest username="mike",realm="sqalab.com",nonce="40da3b76753c8954f01424844614c82531a99e22", uri="sip:pat@10.1.70.249:5060",response="8bea621485a6105b571868763820f95f",algorithm=md5 Content-Length: 260 P-hint: USRLOC Session Description Protocol Session Description Protocol Version (v): 0 Owner/Creator, Session Id (o): ZIP4x4-mike-1.1.65 0 0 IN IP4 10.1.71.131 Session Name (s): zultys media Connection Information (c): IN IP4 10.1.71.131 Time Description, active time (t): 0 0 Media Description, name and address (m): audio 20000 RTP/AVP 0 8 18 101 Media Attribute (a): rtpmap:0 PCMU/8000 Media Attribute (a): rtpmap:8 PCMA/8000 Media Attribute (a): rtpmap:18 G729/8000 Media Attribute (a): rtpmap:101 telephone-event/8000 Media Attribute (a): fmtp:101 0-15 Media Attribute (a): ptime:20 |
| 15. | Internet Protocol, Src Addr: 10.1.71.133 (10.1.71.133), Dst Addr: 10.1.70.249 (10.1.70.249) User Datagram Protocol, Src Port: 5060 (5060), Dst Port: 5060 (5060) Session Initiation Protocol Status line: SIP/2.0 100 Trying Message Header From: "mike" <sip:mike@10.1.70.249:5060>;tag=zip4x4_810445264-42 To: "pat" <sip:pat@10.1.70.249:5060>;tag=zip4x4_3013951804-42 Call-ID: 810444843-42 CSeq: 2 INVITE Via: SIP/2.0/UDP 10.1.70.249;branch=z9hG4bKa798.0647c7d.0,SIP/2.0/UDP 10.1.71.131:5060 Record-Route: <sip:pat@10.1.70.249;ftag=zip4x4_810445264-42;lr=on> Contact: pat <sip:pat@10.1.71.133:5060> Content-Length: 0 |
| 16. | Internet Protocol, Src Addr: 10.1.71.133 (10.1.71.133), Dst Addr: 10.1.70.249 (10.1.70.249) User Datagram Protocol, Src Port: 5060 (5060), Dst Port: 5060 (5060) Session Initiation Protocol Status line: SIP/2.0 180 Ringing Message Header From: "mike" <sip:mike@10.1.70.249:5060>;tag=zip4x4_810445264-42 To: "pat" <sip:pat@10.1.70.249:5060>;tag=zip4x4_3013951804-42 Call-ID: 810444843-42 CSeq: 2 INVITE Via: SIP/2.0/UDP 10.1.70.249;branch=z9hG4bKa798.0647c7d.0,SIP/2.0/UDP 10.1.71.131:5060 Record-Route: <sip:pat@10.1.70.249;ftag=zip4x4_810445264-42;lr=on> Contact: pat <sip:pat@10.1.71.133:5060> Content-Length: 0 |
| 17. | Internet Protocol, Src Addr: 10.1.70.249 (10.1.70.249), Dst Addr: 10.1.71.131 (10.1.71.131) User Datagram Protocol, Src Port: 5060 (5060), Dst Port: 5060 (5060) Session Initiation Protocol Status line: SIP/2.0 180 Ringing Message Header From: "mike" <sip:mike@10.1.70.249:5060>;tag=zip4x4_810445264-42 To: "pat" <sip:pat@10.1.70.249:5060>;tag=zip4x4_3013951804-42 Call-ID: 810444843-42 CSeq: 2 INVITE Via: SIP/2.0/UDP 10.1.71.131:5060 Record-Route: <sip:pat@10.1.70.249;ftag=zip4x4_810445264-42;lr=on> Contact: pat <sip:pat@10.1.71.133:5060> Content-Length: 0 |
| 18. | Internet Protocol, Src Addr: 10.1.71.133 (10.1.71.133), Dst Addr: 10.1.70.249 (10.1.70.249) User Datagram Protocol, Src Port: 5060 (5060), Dst Port: 5060 (5060) Session Initiation Protocol Status line: SIP/2.0 200 OK Message Header From: "mike" <sip:mike@10.1.70.249:5060>;tag=zip4x4_810445264-42 To: "pat" <sip:pat@10.1.70.249:5060>;tag=zip4x4_3013951804-42 Call-ID: 810444843-42 CSeq: 2 INVITE Via: SIP/2.0/UDP 10.1.70.249;branch=z9hG4bKa798.0647c7d.0,SIP/2.0/UDP 10.1.71.131:5060 Record-Route: <sip:pat@10.1.70.249;ftag=zip4x4_810445264-42;lr=on> Contact: pat <sip:pat@10.1.71.133:5060> Allow: INVITE,BYE,CANCEL,ACK,OPTIONS,REGISTER,NOTIFY,MESSAGE,REFER Content-Type: application/sdp Content-Length: 209 Session Description Protocol Session Description Protocol Version (v): 0 Owner/Creator, Session Id (o): ZIP4x4-pat-1.1.65 0 0 IN IP4 10.1.71.133 Session Name (s): zultys media Connection Information (c): IN IP4 10.1.71.133 Time Description, active time (t): 0 0 Media Description, name and address (m): audio 20000 RTP/AVP 0 101 Media Attribute (a): rtpmap:0 PCMU/8000 Media Attribute (a): ptime:20 Media Attribute (a): rtpmap:101 telephone-event/8000 Media Attribute (a): fmtp:101 0-15 |
| 19. | Internet Protocol, Src Addr: 10.1.70.249 (10.1.70.249), Dst Addr: 10.1.71.131 (10.1.71.131) User Datagram Protocol, Src Port: 5060 (5060), Dst Port: 5060 (5060) Session Initiation Protocol Status line: SIP/2.0 200 OK Message Header From: "mike" <sip:mike@10.1.70.249:5060>;tag=zip4x4_810445264-42 To: "pat" <sip:pat@10.1.70.249:5060>;tag=zip4x4_3013951804-42 Call-ID: 810444843-42 CSeq: 2 INVITE Via: SIP/2.0/UDP 10.1.71.131:5060 Record-Route: <sip:pat@10.1.70.249;ftag=zip4x4_810445264-42;lr=on> Contact: pat <sip:pat@10.1.71.133:5060> Allow: INVITE,BYE,CANCEL,ACK,OPTIONS,REGISTER,NOTIFY,MESSAGE,REFER Content-Type: application/sdp Content-Length: 209 Session Description Protocol Session Description Protocol Version (v): 0 Owner/Creator, Session Id (o): ZIP4x4-pat-1.1.65 0 0 IN IP4 10.1.71.133 Session Name (s): zultys media Connection Information (c): IN IP4 10.1.71.133 Time Description, active time (t): 0 0 Media Description, name and address (m): audio 20000 RTP/AVP 0 101 Media Attribute (a): rtpmap:0 PCMU/8000 Media Attribute (a): ptime:20 Media Attribute (a): rtpmap:101 telephone-event/8000 Media Attribute (a): fmtp:101 0-15 |
| 20. | Internet Protocol, Src Addr: 10.1.71.131 (10.1.71.131), Dst Addr: 10.1.70.249 (10.1.70.249) User Datagram Protocol, Src Port: 5060 (5060), Dst Port: 5060 (5060) Session Initiation Protocol Request line: ACK sip:pat@10.1.70.249;ftag=zip4x4_810445264-42;lr=on SIP/2.0 Message Header From: "mike" <sip:mike@10.1.70.249:5060>;tag=zip4x4_810445264-42 To: "pat" <sip:pat@10.1.70.249:5060>;tag=zip4x4_3013951804-42 Call-ID: 810444843-42 CSeq: 2 ACK Via: SIP/2.0/UDP 10.1.71.131:5060 Contact: mike <sip:mike@10.1.71.131:5060> Route: <sip:pat@10.1.71.133:5060> Proxy-Authorization: Digest username="mike",realm="sqalab.com",nonce="40da3b76753c8954f01424844614c82531a99e22", uri="sip:pat@10.1.70.249:5060",response="8bea621485a6105b571868763820f95f",algorithm=md5 Content-Length: 0 |
| 21. | Internet Protocol, Src Addr: 10.1.70.249 (10.1.70.249), Dst Addr: 10.1.71.133 (10.1.71.133) User Datagram Protocol, Src Port: 5060 (5060), Dst Port: 5060 (5060) Session Initiation Protocol Request line: ACK sip:pat@10.1.71.133:5060 SIP/2.0 Message Header Max-Forwards: 10 Record-Route: <sip:pat@10.1.70.249;ftag=zip4x4_810445264-42;lr=on> From: "mike" <sip:mike@10.1.70.249:5060>;tag=zip4x4_810445264-42 To: "pat" <sip:pat@10.1.70.249:5060>;tag=zip4x4_3013951804-42 Call-ID: 810444843-42 CSeq: 2 ACK Via: SIP/2.0/UDP 10.1.70.249;branch=0 Via: SIP/2.0/UDP 10.1.71.131:5060 Contact: mike <sip:mike@10.1.71.131:5060> Proxy-Authorization: Digest username="mike",realm="sqalab.com",nonce="40da3b76753c8954f01424844614c82531a99e22", uri="sip:pat@10.1.70.249:5060",response="8bea621485a6105b571868763820f95f",algorithm=md5 Content-Length: 0 |
| 22. | Internet Protocol, Src Addr: 10.1.71.131 (10.1.71.131), Dst Addr: 10.1.70.249 (10.1.70.249) User Datagram Protocol, Src Port: 5060 (5060), Dst Port: 5060 (5060) Session Initiation Protocol Request line: BYE sip:pat@10.1.70.249;ftag=zip4x4_810445264-42;lr=on SIP/2.0 Message Header From: "mike" <sip:mike@10.1.70.249:5060>;tag=zip4x4_810445264-42 To: "pat" <sip:pat@10.1.70.249:5060>;tag=zip4x4_3013951804-42 Call-ID: 810444843-42 CSeq: 3 BYE Via: SIP/2.0/UDP 10.1.71.131:5060 Route: <sip:pat@10.1.71.133:5060> Proxy-Authorization: Digest username="mike",realm="sqalab.com",nonce="40da3b76753c8954f01424844614c82531a99e22", uri="sip:pat@10.1.70.249:5060",response="8bea621485a6105b571868763820f95f",algorithm=md5 Content-Length: 0 |
| 23. | Internet Protocol, Src Addr: 10.1.70.249 (10.1.70.249), Dst Addr: 10.1.71.133 (10.1.71.133) User Datagram Protocol, Src Port: 5060 (5060), Dst Port: 5060 (5060) Session Initiation Protocol Request line: BYE sip:pat@10.1.71.133:5060 SIP/2.0 Message Header Max-Forwards: 10 Record-Route: <sip:pat@10.1.70.249;ftag=zip4x4_810445264-42;lr=on> From: "mike" <sip:mike@10.1.70.249:5060>;tag=zip4x4_810445264-42 To: "pat" <sip:pat@10.1.70.249:5060>;tag=zip4x4_3013951804-42 Call-ID: 810444843-42 CSeq: 3 BYE Via: SIP/2.0/UDP 10.1.70.249;branch=z9hG4bKb798.36ab5293.0 Via: SIP/2.0/UDP 10.1.71.131:5060 Proxy-Authorization: Digest username="mike",realm="sqalab.com",nonce="40da3b76753c8954f01424844614c82531a99e22", uri="sip:pat@10.1.70.249:5060",response="8bea621485a6105b571868763820f95f",algorithm=md5 Content-Length: 0 |
| 24. | Internet Protocol, Src Addr: 10.1.71.133 (10.1.71.133), Dst Addr: 10.1.70.249 (10.1.70.249) User Datagram Protocol, Src Port: 5060 (5060), Dst Port: 5060 (5060) Session Initiation Protocol Status line: SIP/2.0 200 OK Message Header From: "mike" <sip:mike@10.1.70.249:5060>;tag=zip4x4_810445264-42 To: "pat" <sip:pat@10.1.70.249:5060>;tag=zip4x4_3013951804-42 Call-ID: 810444843-42 CSeq: 3 BYE Via: SIP/2.0/UDP 10.1.70.249;branch=z9hG4bKb798.36ab5293.0,SIP/2.0/UDP 10.1.71.131:5060 Record-Route: <sip:pat@10.1.70.249;ftag=zip4x4_810445264-42;lr=on> Contact: pat <sip:pat@10.1.71.133:5060> Allow: INVITE,BYE,CANCEL,ACK,OPTIONS,REGISTER,NOTIFY,MESSAGE,REFER Content-Length: 0 |
| 25. | Internet Protocol, Src Addr: 10.1.70.249 (10.1.70.249), Dst Addr: 10.1.71.131 (10.1.71.131) User Datagram Protocol, Src Port: 5060 (5060), Dst Port: 5060 (5060) Session Initiation Protocol Status line: SIP/2.0 200 OK Message Header From: "mike" <sip:mike@10.1.70.249:5060>;tag=zip4x4_810445264-42 To: "pat" <sip:pat@10.1.70.249:5060>;tag=zip4x4_3013951804-42 Call-ID: 810444843-42 CSeq: 3 BYE Via: SIP/2.0/UDP 10.1.71.131:5060 Record-Route: <sip:pat@10.1.70.249;ftag=zip4x4_810445264-42;lr=on> Contact: pat <sip:pat@10.1.71.133:5060> Allow: INVITE,BYE,CANCEL,ACK,OPTIONS,REGISTER,NOTIFY,MESSAGE,REFER Content-Length: 0 |

# Call Conference

There is no special signaling in SIP for a conference. Conferences are made up of multiple individual calls. A conference is made by a device that can properly mix the RTP streams. For example, a Zultys ZIP 4x4 phone can mix up to a 5 way conference locally on the phone. The conference is made up of individual inbound out outbound calls.